remote sensing

Article

The VIIRS Day/Night Band: A Flicker Meter in Space?

Christopher D. Elvidge 1,* , Mikhail Zhizhin 1,2 , David Keith 3, Steven D. Miller 4, Feng Chi Hsu 1, Tilottama Ghosh 1, Sharolyn J. Anderson 5, Christian K. Monrad 6, Morgan Bazilian 7, Jay Taneja 8, Paul C. Sutton 9, John Barentine 10, William S. Kowalik 11, Christopher C. M. Kyba 12 , Dee W. Pack 13 and Dorit Hammerling 14

1 Earth Observation Group, Payne Institute for Public Policy, Colorado School of Mines, Golden, CO 80401,

USA; mzhizhin@mines.edu (M.Z.); fengchihsu@mines.edu (F.C.H.); tghosh@mines.edu (T.G.)

2 Russian Space Research Institute, Moscow 117997, Russia

3 WSP Global Inc., Montreal, QC H3H 1P9, Canada; david.keith@wsp.com

4 Cooperative Institute on Research on the Atmosphere, Colorado State University, Fort Collins, CO 80521,

USA; steven.miller@colostate.edu

5 National Park Service—Natural Sounds and Night Skies Team, Fort Collins, CO 80525, USA;

sharolyn_anderson@nps.gov

6 Monrad Engineering, Inc., Tucson, AZ 85719, USA; chrismonrad@monradengineeringinc.com

7 Payne Institute for Public Policy, Colorado School of Mines, Golden, CO 80401, USA; mbazilian@mines.edu

8 Electrical and Computer Engineering, University of Massachusetts, Amherst, MA 01003, USA;

jtaneja@umass.edu

9 Department of Geography, University of Denver, Denver, CO 80210, USA; paul.sutton@du.edu

10 Dark Sky Consulting, LLC, Tucson, AZ 85730, USA; john@darkskyconsulting.com

11 Oregon Chapter of the International Dark-Sky Association, Bend, OR 97709, USA; wsk412@sbcglobal.net

12 GFZ German Research Centre for Geosciences, Telegrafenberg, 14473 Potsdam, Germany;

Citation: Elvidge, C.D.; Zhizhin, M.; Keith, D.; Miller, S.D.; Hsu, F.C.; Ghosh, T.; Anderson, S.J.; Monrad, C.K.; Bazilian, M.; Taneja, J.; et al. The VIIRS Day/Night Band: A Flicker Meter in Space? Remote Sens. 2022, 14, 1316. https://doi.org/10.3390/ rs14061316

Academic Editors: Ran Goldblatt, Steven Louis Rubinyi and

Hogeun Park

Received: 17 January 2022

Accepted: 4 March 2022

Published: 9 March 2022

Publisher’s Note: MDPI stays neutral with regard to jurisdictional claims in published maps and institutional affil iations.

Copyright: © 2022 by the authors. Licensee MDPI, Basel, Switzerland. This article is an open access article distributed under the terms and conditions of the Creative Commons Attribution (CC BY) license (https:// creativecommons.org/licenses/by/

4.0/).

kyba@gfz-potsdam.de

13 Remote Sensing Department, Aerospace Corporation, El Segundo, CA 90245, USA; dee.w.pack@aero.org 14 Department of Applied Mathematics and Statistics, Colorado School of Mines, Golden, CO 80401, USA; hammerling@mines.edu

* Correspondence: celvidge@mines.edu

Abstract: The VIIRS day/night band (DNB) high gain stage (HGS) pixel effective dwell time is in the range of 2–3 milliseconds (ms), which is about one third of the flicker cycle present in lighting powered by alternating current. Thus, if flicker is present, it induces random fluctuations in nightly DNB radiances. This results in increased variance in DNB temporal profiles. A survey of flicker characteristics conducted with high-speed camera data collected on a wide range of individual luminaires found that the flicker is most pronounced in high-intensity discharge (HID) lamps, such as high- and low-pressure sodium and metal halides. Flicker is muted, but detectable, in incandescent luminaires. Modern light-emitting diodes (LEDs) and fluorescent lights are often nearly flicker-free, thanks to high-quality voltage smoothing. DNB pixel footprints are about half a square kilometer and can contain vast numbers of individual luminaires, some of which flicker, while others do not. If many of the flickering lights are drawing from a common AC supplier, the flicker can be synchronized and leave an imprint on the DNB temporal profile. In contrast, multiple power supplies will throw the flickering out of synchronization, resulting in a cacophony with less radiance fluctuation. The examination of DNB temporal profiles for locations before and after the conversion of high-intensity discharge (HID) to LED streetlight conversions shows a reduction in the index of dispersion, calculated by dividing the annual variance by the mean. There are a number of variables that contribute to radiance variations in the VIIRS DNB, including the view angle, cloud optical thickness, atmospheric variability, snow cover, lunar illuminance, and the compilation of temporal profiles using pixels whose footprints are not perfectly aligned. It makes sense to adjust the DNB radiance for as many of these extraneous effects as possible. However, none of these adjustments will reduce the radiance instability introduced by flicker. Because flicker is known to affect organisms, including humans, the development of methods to detect and rate the strength of flickering from space will open up new areas of research on the biologic impacts of artificial lighting. Over time, there is a trend towards the reduction of flicker in outdoor lighting through the replacement of HID with

Remote Sens. 2022, 14, 1316. https://doi.org/10.3390/rs14061316 https://www.mdpi.com/journal/remotesensing

Remote Sens. 2022, 14, x FOR PEER REVIEW 2 of 36  Remote Sens. 2022, 14, 1316 2 of 33

the brightness and steadiness of outdoor lighting can be analyzed with VIIRS DNB temporal pro

low-flicker LED sources. This study indicates that the effects of LED conversions on the brightness

files.

and steadiness of outdoor lighting can be analyzed with VIIRS DNB temporal profiles.

Keywords: VIIRS; DNB; flicker; artificial lighting

Keywords: VIIRS; DNB; flicker; artificial lighting

1. Introduction

1. Introduction

A “flicker meter” is a device capable of recording and characterizing the brightness

A “flicker meter” is a device capable of recording and characterizing the brightness

cycling of lighting powered by alternating current (AC) [1]. In AC, the electrons flow back

cycling of lighting powered by alternating current (AC) [1]. In AC, the electrons flow back

and forth at a well-regulated frequency. Mass distribution power supplies around the

and forth at a well-regulated frequency. Mass distribution power supplies around the

world operate at either 50 or 60 Hertz (cycles per second). AC is frequently converted to

world operate at either 50 or 60 Hertz (cycles per second). AC is frequently converted to

direct current (DC) to provide a more stable power supply to devices and appliances, in a

direct current (DC) to provide a more stable power supply to devices and appliances, in a

process called “rectification”. The full-wave rectification (Figure 1) of a 60 Hz source re

process called “rectification”. The full-wave rectification (Figure 1) of a 60 Hz source results

sults in lights flickering at 120 Hz, but with half the amplitude of the original alternating

in lights flickering at 120 Hz, but with half the amplitude of the original alternating current.

current. Further reductions in the amplitude of flicker can be achieved by voltage smooth

Further reductions in the amplitude of flicker can be achieved by voltage smoothing with

ing with capacitors (common in fluorescent lights) and “constant current” drivers [2] used

capacitors (common in fluorescent lights) and “constant current” drivers [2] used to power

to power large-wattage LED sources (Figure 1).

large-wattage LED sources (Figure 1).

Figure 1. Voltage versus time patterns for alternating current, full-wave rectification, smoothing

Figure 1. Voltage versus time patterns for alternating current, full-wave rectification, smoothing

accomplished with a capacitor, and LED “constant-current”.

accomplished with a capacitor, and LED “constant-current”.

The “critical flicker frequency” [3] or frame rate of human vision is variable but is

The “critical flicker frequency” [3] or frame rate of human vision is variable but is

placed in the range of 3 to 70 Hz [4]. It is well established that lighting flicker within

placed in the range of 3 to 70 Hz [4]. It is well established that lighting flicker within the

the range of visual perception can induce neurological symptoms like headaches, loss of

range of visual perception can induce neurological symptoms like headaches, loss of at

attention and visual acuity, irritability, and in some cases, epileptic seizures [5,6]. Most

tention and visual acuity, irritability, and in some cases, epileptic seizures [5,6]. Most

Remote Sens. 2022, 14, 1316 3 of 33

modern luminaires have been engineered to minimize flicker below 50 Hz. However, many

light sources flicker in the range of 100 to 120 Hz, which is above the range of conscious

perception to the human eye.

The effects of 100–120 Hz flicker on humans remains an active research area. One

recent study found that humans are subtly affected by lighting flicker up to 500 Hz [7]. Exposure to flicker at rates above 70 Hz for long periods of time can result in neurological

symptoms such as induced photosensitivity [8], chronic migraine and fatigue, and anxiety,

depression, and irritability [9]. There is a growing body of evidence that lighting flicker

adversely affects other organisms [10]. The satellite detection and characterization of

flickering lights would open up a range of new research opportunities in the field of

anthropogenic impacts on many organisms, including humans.

To fully resolve the flicker pattern requires temporal sampling that is nearly ten times

shorter than the flicker rate being measured [11]. Thus, for 120 Hz flickering, the flicker

meter, as one of its design criteria, should measure the brightness of the light at 1200 Hz. This corresponds to a sampling rate of 8.3 × 10−4seconds (s). According to Table 3.3-6 on

page 52 of the VIIRS geolocation Algorithm Theoretical Basis Document (ATBD) [12], the

VIIRS DNB pixel dwell time ranges from 2.53 × 10−4s at nadir to 4.22 × 10−5s at the edge

of scan. If this is true, the DNB pixel dwell time is shorter than the minimum acceptable

sampling interval recommended for characterizing 120 Hz flickering.

The standard flicker meter collects many cycles in a single collection session to fully

resolve the temporal pattern of the flicker of an individual light. In contrast, the VIIRS

DNB collects once or twice per night across many years. At 742 m on a side, DNB pixels

may contain hundreds of individual lights of various types. Radiance calibrated VIIRS

DNB temporal profiles can be assembled, filtered to remove flawed data, adjusted for a

range of radiance contributors such as moonlight, and analyzed for events such as power

outages or cycling. Is it also possible to discern differences in AC flicker patterns within

VIIRS temporal profiles?

In this paper, we clarify the VIIRS DNB pixel dwell time specifically for the nighttime

collection mode, define the flicker characteristics for the major types of lighting, and

examine VIIRS DNB temporal profiles for evidence of flicker.

2. Materials and Methods

2.1. Calculation of DNB Pixel Dwell Times for High Gain Stage Data

The VIIRS DNB focal plane assembly contains four detector arrays (Figure 2), which

are swept across the earth to construct 16 lines of DNB pixels on each scan [12]. DNB pixel

footprints on the earth’s surface are maintained at almost 742 m on a side from nadir to edge

of scan by the onboard aggregation of different numbers of adjacent detectors in both the

track and scan directions. The number of detectors aggregated is largest at nadir and drops

gradually, moving toward the edge of scan in fixed patterns, referred to as aggregation

zones. Pixels from the first aggregation zone, near nadir, are constructed using 66 detectors

in the scan direction and 42 in the track direction (Figure 2). In aggregation zone 32—at the

edge of scan—pixels are formed using 11 detectors in the scan direction and 20 detectors in

the track direction (Figure 2). All four DNB detector arrays have 672 detectors in the track

direction to afford the exact number of detectors required to form the aggregation zone

1 pixels (16 × 42 = 672). Each of the DNB detector arrays are 672 detectors high but vary

in width based on the amplification requirements to image clouds from daylit to moonlit

conditions. Under daylit conditions, no amplification is required, so the low gain stage

(LGS) focal plane array is only one detector wide. The signal amplification is achieved

by accumulating charge from multiple samples of the same sub-pixels—a process known

as time delay and integration (TDI). The medium gain stage (MGS), designed for cloud

imaging in the transition from day to night at dawn and dusk, has three parallel rows of

detectors for collecting data with a TDI of three. The highest level of signal amplification

comes from the high gain stage (HGS), which is 250 detectors wide. The HGS arrays collect

Remote Sens. 2022, 14, 1316 4 of 33 Remote Sens. 2022, 14, x FOR PEER REVIEW 4 of 36

data with a TDI of 250. The HGS is designed to collect at night and is responsible for

arrays collect data with a TDI of 250. The HGS is designed to collect at night and is re

recording radiances from electric lighting.

sponsible for recording radiances from electric lighting.

Figure 2. Schematic showing the four DNB detector arrays and the aggregation of charge from

Figure 2. Schematic showing the four DNB detector arrays and the aggregation of charge from var

variably sized blocks of detectors to maintain near-constant 742 × 742 m ground footprints from

iably sized blocks of detectors to maintain near-constant 742 × 742 m ground footprints from nadir

nadir to edge of scan. LGS = low gain stage, optimized for daytime collections. MGS = medium

to edge of scan. LGS = low gain stage, optimized for daytime collections. MGS = medium gain stage,

optimized for collection across the terminator during the transition from day to night. HGS = high

gain stage, optimized for collection across the terminator during the transition from day to night.

gain stage, optimized for nighttime imaging.

HGS = high gain stage, optimized for nighttime imaging.

There are two HGS detector arrays, referred to as HGS-A and HGS-B. This redundancy

There are two HGS detector arrays, referred to as HGS-A and HGS-B. This redun

is designed to filter out high energy particle detections, which are common when the

dancy is designed to filter out high energy particle detections, which are common when

spacecraft flies through the South Atlantic Anomaly (SAA), which is centered near Rio de

the spacecraft flies through the South Atlantic Anomaly (SAA), which is centered near Rio

Janeiro, Brazil. An onboard algorithm compares the signal from the same ground pixel

de Janeiro, Brazil. An onboard algorithm compares the signal from the same ground pixel

from HGS-A and HGS-B. If there is a large difference between the two, the brighter pixel is

from HGS-A and HGS-B. If there is a large difference between the two, the brighter pixel

dropped, and the pixel’s recorded radiance only comes from one of the two HGS detector

is dropped, and the pixel’s recorded radiance only comes from one of the two HGS detec

arrays. If the difference is modest, the two HGS signals are averaged.

tor arrays. If the difference is modest, the two HGS signals are averaged.

The detector array sampling rate of the DNB is 260.6 KHz [12], corresponding to

The detector array sampling rate of the DNB is 260.6 KHz [12], corresponding to

3.83729 × 10−3 milliseconds (ms) per detector. The DNB integration table in the ATBD [12]

3.83729 × 10−3 milliseconds (ms) per detector. The DNB integration table in the ATBD [12]

calculates DNB pixel collection times by multiplying the subpixels in the scan direction

calculates DNB pixel collection times by multiplying the subpixels in the scan direction

by 3.83729 × 10−3s. These numbers are correct for the LGS, which has no TDI, and

by 3.83729 × 10−3 s. These numbers are correct for the LGS, which has no TDI, and accu

accumulating all 66 along-scan detectors (aggregation zone 1) yields 2.53 × 10−4s. To add

mulating all 66 along-scan detectors (aggregation zone 1) yields 2.53 × 10−4 s. To add in the

in the TDI to the DNB pixel HGS collection time, an oversample factor of 250 is applied

TDI to the DNB pixel HGS collection time, an oversample factor of 250 is applied to each

to each detector in the along-scan direction (Figure 3). Thus, at nadir, the HGS-A pixel

detector in the along-scan direction (Figure 3). Thus, at nadir, the HGS-A pixel collection

collection time is (66 + 250) times 3.83729 × 10−3 ms = 1.213 ms. To this, we add the gap

time is (66 + 250) times 3.83729 × 10−3 ms = 1.213 ms. To this, we add the gap time between

time between HGS-A and HGS-B, which is estimated to be 0.25 ms, and then add the

HGS-A and HGS-B, which is estimated to be 0.25 ms, and then add the time required to

time required to collect HGS-B, which is the same (1.213 ms) as HGS-A. Thus, the total

collect HGS-B, which is the same (1.213 ms) as HGS-A. Thus, the total integration time for

integration time for a HGS DNB pixel at nadir is 2.67 ms, while for aggregation zone 32, it

a HGS DNB pixel at nadir is 2.67 ms, while for aggregation zone 32, it is 2.29 ms.

is 2.29 ms.

Remote Sens. Remote Sens. 2022, 14, 1316 5 of 33 2022, 14, x FOR PEER REVIEW 5 of 36

Figure 3. Timeline for the VIIRS DNB HGS pixel aggregation.

Figure 3. Timeline for the VIIRS DNB HGS pixel aggregation.

It is important to note that the four DNB detector arrays collect photons and accumu

It is important to note that the four DNB detector arrays collect photons and accumu

late charge continuously during the scan across the earth, but that the zone of active pixel

late charge continuously during the scan across the earth, but that the zone of active pixel

formation gradually tapers from nadir to edge of scan (Figure 4) to maintain the 742 m

formation gradually tapers from nadir to edge of scan (Figure 4) to maintain the 742 m

DNB pixel size. At nadir, all 672 detectors are read out to form pixels. At the edge of scan,

DNB pixel size. At nadir, all 672 detectors are read out to form pixels. At the edge of scan,

the number of detectors actively involved in pixel formation drops to 320. Similar tapering

the number of detectors actively involved in pixel formation drops to 320. Similar tapering

occurs on the LGS and MGS arrays. At any one instant, the HGS detector arrays collect

occurs on the LGS and MGS arrays. At any one instant, the HGS detector arrays collect

data for a number for DNB pixels. This number can be calculated as 250 divided by the

data for a number for DNB pixels. This number can be calculated as 250 divided by the

number of sub-pixels in the scan direction. At nadir, this is 250/66 = 2.79 pixels and at the

number of sub-pixels in the scan direction. At nadir, this is 250/66 = 2.79 pixels and at the

edge of scan, 250/11 = 22.72 pixels. Onboard, the DNB evaluates the pixel voltages from

edge of scan, 250/11 = 22.72 pixels. Onboard, the DNB evaluates the pixel voltages from

HGS, MGS, and LGS to record the highest unsaturated voltage. Pixels with bright surface

HGS, MGS, and LGS to record the highest unsaturated voltage. Pixels with bright surface

lighting will have a usable signal in the HGS but remain undetectable in the MGS and LGS.

lighting will have a usable signal in the HGS but remain undetectable in the MGS and

Conversely, daylit clouds will have a usable voltage from the LGS, while the HGS signal

LGS. Conversely, daylit clouds will have a usable voltage from the LGS, while the HGS

will be saturated. This arrangement makes it possible for the DNB to instantly adjust to

signal will be saturated. This arrangement makes it possible for the DNB to instantly ad

abrupt changes in brightness, such as transits across the dawn–dusk lines.

just to abrupt changes in brightness, such as transits across the dawn–dusk lines.

As an interesting side note, we expected that the HGS noise filtering would elimi

nate the South Atlantic Anomaly high-energy particle detections commonly found in the

low-light imaging data collected by the VIIRS predecessor: the U.S. Air Force Defense

Meteorological Satellite Program (DMSP) Operational Linescan System (OLS). The SAA

affected pixels have high radiance levels in unexpected places, such as the Atlantic Ocean

between Africa and South America. Their numbers appear unchanged between OLS and

VIIRS. Several years ago, we conducted an experiment where HGS-A and HGS-B were

brought down intact from the satellite along with the normal DNB data stream using

one of the VIIRS calibration modes. The HGS-A and HGS-B could not be brought down

simultaneously, so they were brought down in alternating granules collected over the

Atlantic Ocean south of the equator—a zone famous for SAA detections. We found that

SAA hits in both the HGS-A and HGS-B, but these were absent from the normal DNB data.

However, the normal DNB data from those granules had a different set of SAA detection

pixels. Our conclusion was that the HGS noise filtering is working as designed. Therefore,

Remote Sens. 2022, 14, 1316 6 of 33

Remote Sens. 2022, 14, x FOR PEER REVIEW 6 of 36

the SAA detections present in the DNB data stream must be coming from the MGS or LGS,

which lack the noise filtering.

Figure 4. The active zone of DNB pixel formation tapers from 672 detectors in the track direction at

Figure 4. The active zone of DNB pixel formation tapers from 672 detectors in the track direction at

nadir to 320 detectors at the edge of scan.

nadir to 320 detectors at the edge of scan.

2.2. Collection of High-Speed Camera Data of Individual Luminaires

As an interesting side note, we expected that the HGS noise filtering would eliminate

High-speed camera (HSC) data and spectral signatures were collected for the set of

the South Atlantic Anomaly high-energy particle detections commonly found in the low

individual luminaires listed in Table 1. Most of the data collections were made with a Sony

light imaging data collected by the VIIRS predecessor: the U.S. Air Force Defense Meteor

Cyber-Shot DSC-RX100 V 20.1 operated at 1000 frames per second. A second camera was

ological Satellite Program (DMSP) Operational Linescan System (OLS). The SAA-affected

used to collect data from a hovering DJI Mavic Air 2S drone in Moscow using the built-in

pixels have high radiance levels in unexpected places, such as the Atlantic Ocean between

camera running at 120 frames per second. We also experimented with the collection of

Africa and South America. Their numbers appear unchanged between OLS and VIIRS.

flicker data on the panoramic views of city lights with a night vision photomultiplier tube

Several years ago, we conducted an experiment where HGS-A and HGS-B were brought

(PMT) adapter (generation 3+ auto-gated white phosphor AGM PVS-14) replacing the fore

down intact from the satellite along with the normal DNB data stream using one of the

optics of the Sony camera. Both the original Sony camera and PMT-equipped Sony camera

VIIRS calibration modes. The HGS-A and HGS-B could not be brought down simultane

were tested for the presence of flicker or other aberrations with high-speed camera data

ously, so they were brought down in alternating granules collected over the Atlantic

collected on the moon as a flicker-free light source in the night sky.

Ocean south of the equator—a zone famous for SAA detections. We found that SAA hits

The MP4 files collected of the luminaires are available via the links listed in the

in both the HGS-A and HGS-B, but these were absent from the normal DNB data. How

supplemental material. Temporal profiles were produced from rectangular subsets of the

ever, the normal DNB data from those granules had a different set of SAA detection pixels.

video frames centered on the light sources. A brightness threshold was applied to each

Our conclusion was that the HGS noise filtering is working as designed. Therefore, the

frame to minimize the inclusion of background into the temporal profiles, which are also

SAA detections present in the DNB data stream must be coming from the MGS or LGS,

available in the supplemental material.

which lack the noise filtering.

Luminaire spectra were collected with a BWTek BRC112E spectrometer covering the

400 to 1000 nanometer range. The spectrometer was mounted to record the luminaires’

2.2. Collection of High-Speed Camera Data of Individual Luminaires

reflectance from a Spectralon panel. Like fingerprints, the spectra were used to confirm the

High-speed camera (HSC) data and spectral signatures were collected for the set of

lighting types based on previously published spectral signatures of lights [13].

individual luminaires listed in Table 1. Most of the data collections were made with a Sony

Cyber-Shot DSC-RX100 V 20.1 operated at 1000 frames per second. A second camera was

used to collect data from a hovering DJI Mavic Air 2S drone in Moscow using the built-in

camera running at 120 frames per second. We also experimented with the collection of

flicker data on the panoramic views of city lights with a night vision photomultiplier tube

(PMT) adapter (generation 3+ auto-gated white phosphor AGM PVS-14) replacing the fore

optics of the Sony camera. Both the original Sony camera and PMT-equipped Sony camera

were tested for the presence of flicker or other aberrations with high-speed camera data

collected on the moon as a flicker-free light source in the night sky.

Remote Sens. 2022, 14, 1316 7 of 33

Table 1. Individual luminaires measured.

Luminaire

Type Designation Sample Manufacturer Model Wattage

Metal Halide Ceramic Metal

Halide C0009 Sylvania MetalARC Mp

100/U/MED 100

Metal Halide C0012 EMCO Lighting ERA20-3H

SO-89193 175

Metal Halide C0015 Philips MP400/BU/PS

Kr85 400

Metal Halide

RAB C0016 RAB RAB LMH250PS 250

High Pressure

Sodium HPS C0017 Sylvania LU150/55/ECO 175

HPS C0041 General Electric Lucalox 100

Low Pressure

Sodium LPS C0007 Osram N068 80X Great

Britain 18

Fluorescent 4 foot tube pair C0006 Philips Alto F40T/C50

Sumpreme 40

Fluorescent

ceiling C0035 Philips Compact

Alto-II

F32T8/Tl841 800 series

32

Fluorescent C0028 BFLG 13 13 Compact

Fluorescent C0031 Greenlight 13W/ELS-M/1 13 Compact

Fluorescent C0032 TCP ESN11 11 Incandescent Incandescent C0014

Incandescent C0027 Sylvania Capsylite FL tugsten-halogen 45 Incandescent C0034 Soft White 60 LED LED C0018 PLT Solutions PLT-11557 300 LED 4000 K C0020 LeoTEK Beta 100 LED 3000 K C0022 LeoTEK Beta 100 LED C0026 Lithonia DSXPG 51

iphone C0036 Apple iPhone Xr

MT472LL/A ?

2.3. Collection of Flicker Data for Mixtures of Lights

To investigate the expression of flicker that might be present in VIIRS DNB data collected on pixels having large numbers of individual luminaires, high-speed camera data were collected for a set of scenes containing mixtures of light sources. We speculated that the magnitude of flicker would decline when large numbers of individual luminaires are viewed together. The first collection in the multi-light series was made on a wide range of luminaires from Table 1 placed together on an office table. This included LED streetlights, metal halides, high-pressure sodium, low-pressure sodium, incandescent, and fluorescent lights (Figure 5).

ecte on pixes aving arge numers o iniviua uminaires, ig-spee camera ata

were collected for a set of scenes containing mixtures of light sources. We speculated that

the magnitude of flicker would decline when large numbers of individual luminaires are  viewed together. The first collection in the multi-light series was made on a wide range of  luminaires from Table 1 placed together on an office table. This included LED streetlights,

Remote Sens. 2022, 14, 1316 8 of 33 metal halides, high-pressure sodium, low-pressure sodium, incandescent, and fluorescent  lights (Figure 5).

Figure 5. High-speed camera data were collected on a mixture of luminaires placed together on a

Figure 5. High-speed camera data were collected on a mixture of luminaires placed together on a

table.

table.

Next, we collected high-speed camera data (1000 frames per second) from the ground

Next, we collected high-speed camera data (1000 frames per second) from the ground

for three outdoor scenes containing large numbers of individual luminaires. This in

for three outdoor scenes containing large numbers of individual luminaires. This included:

cluded: (1) Broomfield, Colorado’s civic center (Figure 6), (2) the central zone of Golden,

(1) Broomfield, Colorado’s civic center (Figure 6), (2) the central zone of Golden, Colorado

Colorado (Figure 7), collected from a viewpoint located on Lookout Mountain Road, and

(Figure 7), collected from a viewpoint located on Lookout Mountain Road, and (3) Boulder,

(3) Boulder, Colorado, viewed at a distance from a viewpoint along Highway 36 (Figure

Colorado, viewed at a distance from a viewpoint along Highway 36 (Figure 8). The

8). The panoramic collections of Golden and Boulder cover large portions of each city as

panoramic collections of Golden and Boulder cover large portions of each city as viewed

viewed from an oblique angle. The resulting temporal profiles from Golden and Boulder

from an oblique angle. The resulting temporal profiles from Golden and Boulder cover

cover much larger areas than the individual DNB pixel footprint. The examination of the

much larger areas than the individual DNB pixel footprint. The examination of the MP4

MP4 movies of these collections reveals the presence of a cacophony of sources ranging

movies of these collections reveals the presence of a cacophony of sources ranging from

Remote Sens. 2022, 14, x FOR PEER REVIEW 9 of 36  from lights having prominent flicker to lights having no visually evident flicker.

lights having prominent flicker to lights having no visually evident flicker.

Figure 6. High-speed camera data were collected on a mixture of lights present at the Civic Center

Figure 6. High-speed camera data were collected on a mixture of lights present at the Civic Center in

in Broomfield, Colorado. Also included in the scene was the moon—in the upper left-hand corner—

Broomfield, Colorado. Also included in the scene was the moon—in the upper left-hand corner—

which made it possible to analyze the flicker from the camera itself.

which made it possible to analyze the flicker from the camera itself.

Remote Sens. 2022, 14, 1316 9 of 33 Figure 6. High-speed camera data were collected on a mixture of lights present at the Civic Center  in Broomfield, Colorado. Also included in the scene was the moon—in the upper left-hand corner—

Figure 6. High-speed camera data were collected on a mixture of lights present at the Civic Center  in Broomfield, Colorado. Also included in the scene was the moon—in the upper left-hand corner—

which made it possible to analyze the flicker from the camera itself.

which made it possible to analyze the flicker from the camera itself.

Figure 7. Oblique view of the mixture of lights presents in the central Golden, Colorado area col

Figure 7. Oblique view of the mixture of lights presents in the central Golden, Colorado area collected

Figure 7. Oblique view of the mixture of lights presents in the central Golden, Colorado area col

lected with the high-speed camera. The data collection was from a viewpoint along Lookout Moun

with the high-speed camera. The data collection was from a viewpoint along Lookout Mountain

lected with the high-speed camera. The data collection was from a viewpoint along Lookout Moun

tain Road west of Golden.

Road west of Golden.

tain Road west of Golden.

Figure 8. Panoramic view of Boulder, Colorado, from a viewpoint along Highway 36. High-speed  Figure 8. Panoramic view of Boulder, Colorado, from a viewpoint along Highway 36. High-speed

camera data were collected covering the full sweep of lights visible in Boulder from the viewpoint.

Figure 8. Panoramic view of Boulder, Colorado, from a viewpoint along Highway 36. High-speed

camera data were collected covering the full sweep of lights visible in Boulder from the viewpoint.

camera data were collected covering the full sweep of lights visible in Boulder from the viewpoint.

To further investigate the remote sensing of flicker from the DNB, digital camera data

were collected from a hovering drone carrying a slow-motion camera pointed in the nadir

direction towards the earth. The experiment took place in the suburbs of Moscow, Russia

(55.6321 north, 37.8622 east), at 01:00 local time on 29 November 2021. The vehicle was a

DJI Air 2S drone with a built-in 4K video camera. The data were collected while the drone

hovered 315 m above the earth’s surface, resulting in a footprint of 600 × 600 m—slightly

smaller than the DNB pixel footprint. The nighttime movie of the lights was recorded with

FHD resolution 1920 × 1080, shutter speed 1/120 s, f/2.8 aperture, and sensitivity ISO

3200. The data were collected at 120 frames per second. The scene includes numerous

flickering high-pressure sodium streetlights and parking lot lights, plus strings of white

lights (Figure 9), with some clearly flickering and others with no obvious flickering.

DJI Air 2S drone with a built-in 4K video camera. The data were collected while the drone

hovered 315 m above the earth’s surface, resulting in a footprint of 600 × 600 m—slightly

smaller than the DNB pixel footprint. The nighttime movie of the lights was recorded with

FHD resolution 1920 × 1080, shutter speed 1/120 s, f/2.8 aperture, and sensitivity ISO 3200.

The data were collected at 120 frames per second. The scene includes numerous flickering

Remote Sens. 2022, 14, 1316 10 of 33 high-pressure sodium streetlights and parking lot lights, plus strings of white lights (Fig

ure 9), with some clearly flickering and others with no obvious flickering.

Figure 9. View of lights collected at 120 frames per second from a hovering drone in Moscow, Rus

Figure 9. View of lights collected at 120 frames per second from a hovering drone in Moscow, Russia.

sia.

2.4. Processing of the High-Speed Camera Data

2.4. Processing of the High-Speed Camera Data

The digital data from the high-speed camera MP4 movie frames were filtered to

The digital data from the high-speed camera MP4 movie frames were filtered to re

remove background pixels, which helped to concentrate the signal coming from the lights. move background pixels, which helped to concentrate the signal coming from the lights.

The brightness of the remaining pixels in each frame were averaged and frame step times

The brightness of the remaining pixels in each frame were averaged and frame step times

added. For the Sony camera, the frame steps were in millisecond increments since the

added. For the Sony camera, the frame steps were in millisecond increments since the

camera ran at 1000 frames per second. The time step for the drone camera collection was

camera ran at 1000 frames per second. The time step for the drone camera collection was

8.333 ms, corresponding to 120 frames per second. The result was a set of temporal profiles

8.333 ms, corresponding to 120 frames per second. The result was a set of temporal profiles

that were further analyzed to detect the presence and relative intensity of flicker.

that were further analyzed to detect the presence and relative intensity of flicker.

An autocorrelation function (ACF) was used for the detection of flicker in the HSC data

An autocorrelation function (ACF) was used for the detection of flicker in the HSC

and a determination of the cycling rate in Hz. The ACF rates the similarity of the time series

data and a determination of the cycling rate in Hz. The ACF rates the similarity of the time

with itself for a series of evenly spaced temporal or lag increments [14]. For instance, if the

series with itself for a series of evenly spaced temporal or lag increments [14]. For instance,

data series is broken up into a series of consecutive five ms segments, the ACF calculates

if the data series is broken up into a series of consecutive five ms segments, the ACF cal

the degree of similarity found between the segments. This autocorrelation analysis is

culates the degree of similarity found between the segments. This autocorrelation analysis

performed for all possible lag increments within the time series. The types of detectable

is performed for all possible lag increments within the time series. The types of detectable

temporal phenomena from the ACF analysis are dictated by the temporal increment of the

temporal phenomena from the ACF analysis are dictated by the temporal increment of

data. For instance, annual cycling cannot be analyzed with annual nighttime lights data

the data. For instance, annual cycling cannot be analyzed with annual nighttime lights

but can be analyzed with either nightly radiances [15] or average monthly radiances [16]. data but can be analyzed with either nightly radiances [15] or average monthly radiances

The presence of an AC flicker cycling in the high-speed camera data is indicated when an

[16]. The presence of an AC flicker cycling in the high-speed camera data is indicated

ACF peak exceeds the 95% or 99% confidence level threshold calculated via a z-statistic [17]. when an ACF peak exceeds the 95% or 99% confidence level threshold calculated via a z

For an ACF sample size of 100 points, the 95% confidence threshold is 0.196, and the 99%

statistic [17]. For an ACF sample size of 100 points, the 95% confidence threshold is 0.196,

Remote Sens. 2022, 14, x FOR PEER REVIEW 11 of 36  confidence threshold is 0.258. A flicker rate of 120 Hz is indicated by the presence of a

and the 99% confidence threshold is 0.258. A flicker rate of 120 Hz is indicated by the

primary ACF peak at 8 ms, exceeding the 95% confidence thresholds (Figure 10).

presence of a primary ACF peak at 8 ms, exceeding the 95% confidence thresholds (Figure

10).

Figure 10. An autocorrelation function (ACF) is generated for each high-speed camera temporal

Figure 10. An autocorrelation function (ACF) is generated for each high-speed camera temporal

profile to detect the presence of millisecond-scale cycling. The primary peak is defined as the highest

profile to detect the presence of millisecond-scale cycling. The primary peak is defined as the highest

local max lag value greater than 1 ms and exceeding 95% confidence. The Sony camera collected at

local max lag value greater than 1 ms and exceeding 95% confidence. The Sony camera collected at

1000 frames per second. An ACF lag of 8 ms indicates the presence of 120 Hz flickering.

1000 frames per second. An ACF lag of 8 ms indicates the presence of 120 Hz flickering.

Calculations were made of the percent flicker and flicker index based on the methods

described in the Lighting Handbook 10th Edition [18]; examples are shown in Figure 11.

Note that in order to apply the flicker index to the randomized and actual DNB profiles,

the definition had to be modified. In addition, the index of dispersion is calculated as the

variance divided by the mean [19].

Percent flicker is designed to quantify the amplitude of the flicker and is defined as:

Remote Sens. 2022, 14, 1316 11 of 33

Calculations were made of the percent flicker and flicker index based on the methods

described in the Lighting Handbook 10th Edition [18]; examples are shown in Figure 11.

Note that in order to apply the flicker index to the randomized and actual DNB profiles,

Remote Sens. 2022, 14, x FOR PEER REVIEW 12 of 36  the definition had to be modified. In addition, the index of dispersion is calculated as the

variance divided by the mean [19].

Figure 11. Calculation of percent flicker, flicker index, and index of dispersion.

Figure 11. Calculation of percent flicker, flicker index, and index of dispersion.

2.5. Simulation of DNB Temporal Profiles from the High-Speed Camera Data  Because the VIIRS DNB temporal repeat cycle ranges from hours to days, the flicker  patterns in lighting are sampled randomly. It is impossible to discern the lighting flicker  rates with the ACF methodology using nightly time series of VIIRS DNB temporal pro files. However, we still want to test the flicker indices to evaluate their utility in analyzing  the flicker effects present in DNB temporal profiles. Each DNB scanline has 4064 pixels

Remote Sens. 2022, 14, 1316 12 of 33

Percent flicker is designed to quantify the amplitude of the flicker and is defined as:

Percent Flicker =A − B

A + B× 100

For the high-speed camera temporal profiles, A is the average brightness of the upper

peak, while B is the average brightness of the lower peak. For the simulated and actual

DNB temporal profiles, A and B are the upper and lower limits of the core data range. Outlier removal and detrending may be required to obtain results that can be compared

across samples.

The flicker index is designed to quantify the symmetry of the flicker and is defined as:

Flicker Index =∑S − S

∑ S

where the S is the sample brightness, and S is the mean of sample brightness.

The dispersion index is defined as:

Dispersion Index =var(S)

S

where var(S) is the variance of sample brightness.

As with the percent flicker, outlier removal and trend removal may be advisable prior

to the flicker index and dispersion calculations to yield consistent results.

2.5. Simulation of DNB Temporal Profiles from the High-Speed Camera Data

Because the VIIRS DNB temporal repeat cycle ranges from hours to days, the flicker

patterns in lighting are sampled randomly. It is impossible to discern the lighting flicker

rates with the ACF methodology using nightly time series of VIIRS DNB temporal profiles. However, we still want to test the flicker indices to evaluate their utility in analyzing the

flicker effects present in DNB temporal profiles. Each DNB scanline has 4064 pixels and

the HGS dwell time varies as a function of the sample position. The SNPP orbit processes,

with a 16 day repeat cycle. That means the satellite repeats its orbital tracks every 16 days, with only minor deviations. The repeat cycle, combined with the spacecraft’s precision

orbit control, results in a series of narrowly defined “permissible” sample positions for

any location on the ground. To mimic the variable temporal sampling of the HGS, we

followed the sampling pattern established for a particular mid-latitude location, as shown

in Figure 12. Any location would suffice due to the even mixture of sample positions

over time. The Figure 12 template happens to be from Houston, Texas, and has 1946

cloud-free DNB pixels from 2012 to 2020, binned into 20 vertical sample position columns. In simulating the DNB temporal profile from each high-speed camera temporal profile,

exactly 1946 samples are drawn randomly from the brightness profile using the sample

positions from Figure 12. The randomly drawn sample positions point to the appropriate

DNB temporal aggregation time via a look up table. The simulated DNB temporal profiles

have the same brightness units and range as the original 1000 frame per second profiles

but are randomized. The A and B inflection points used to calculate the percent flicker are

taken to be the upper and lower brightness levels of the data cloud (Figure 11).

.         , ,    -

DNB pixels from 2012 to 2020, binned into 20 vertical sample position columns. In simu

lating the DNB temporal profile from each high-speed camera temporal profile, exactly

1946 samples are drawn randomly from the brightness profile using the sample positions

from Figure 12. The randomly drawn sample positions point to the appropriate DNB tem

poral aggregation time via a look up table. The simulated DNB temporal profiles have the

Remote Sens. 2022, 14, 1316 13 of 33

same brightness units and range as the original 1000 frame per second profiles but are

randomized. The A and B inflection points used to calculate the percent flicker are taken

to be the upper and lower brightness levels of the data cloud (Figure 11).

Remote Sens. 2022, 14, x FOR PEER REVIEW 14 of 36  Figure 12. Sample position columns for the VIIRS DNB temporal profile from a 15-arcsecond grid

Figure 12. Sample position columns for the VIIRS DNB temporal profile from a 15-arcsecond grid

cell in Houston, Texas. These columns define the sampling pattern for simulating the DNB temporal

cell in Houston, Texas. These columns define the sampling pattern for simulating the DNB temporal

profiles from the high-speed camera data for each of the tested luminaires.

The grid cells are divided into three zones to examine the effects of the streetlight

profiles from the high-speed camera data for each of the tested luminaires.

conversions on the index values. First, the grid cells are divided into those with and with

2.6. Examination of VIIRS DNB Temporal Profiles for Flicker Effects

2.6. Examination of VIIRS DNB Temporal Profiles for Flicker Effects

out detectable lighting (Figure 14). Background areas, devoid of detectable lighting, are

To explore the detection of flicker in actual VIIRS data, a nightly DNB temporal pro

defined based on annual average radiance values under one nanowatt/cm2/sr. The lit grid

file grid was constructed for Tucson, Arizona. We generated the profiles at 15-arcsecond

To explore the detection of flicker in actual VIIRS data, a nightly DNB temporal profile

spacing, with 100 cells east–west and 72 cells north–south centered on Tucson (Figure 13).

cells are further divided into sets with and without City of Tucson LED streetlight con

grid was constructed for Tucson, Arizona. We generated the profiles at 15-arcsecond

The DNB profiles were processed to remove sunlit and cloudy data. A satellite zenith an

versions (Figure 15).

spacing, with 100 cells east–west and 72 cells north–south centered on Tucson (Figure 13).

gle normalization was applied to remove view angle effects [15]. A lunar illuminance ad

A set of five Tucson grid cells were selected for more detailed examination and test

The DNB profiles were processed to remove sunlit and cloudy data. A satellite zenith

justment was applied to subtract the reflected moonlight [15]. Cloudy pixels were filtered

ing. This included one grid cell from the background. The other four grid cells were se

out based on the VIIRS cloud mask [16] generated by NOAA [20]. The annual mean, var

angle normalization was applied to remove view angle effects [15]. A lunar illuminance

lected because they show abrupt and dramatic changes in radiance levels, with two from

iance, and dispersion were then calculated for each 15-arcsecond grid cell.

adjustment was applied to subtract the reflected moonlight [15]. Cloudy pixels were

the city’s LED conversion zone and two from areas outside the city’s LED conversion

For decades, Tucson has taken actions to reduce light pollution to limit interference

filtered out based on the VIIRS cloud mask [16] generated by NOAA [20]. The annual

zone. For these five grid cells, the annual percent flicker, flicker index, and index of dis

with several nearby astronomical observatories, such as Kitt Peak National Observatory.

mean, variance, and dispersion were then calculated for each 15-arcsecond grid cell.

persion values were calculated.

In 2016, the City of Tucson began replacing high-pressure sodium streetlights with LED

luminaires [21–23]. The motivation for the streetlight conversions was two-fold. There

was a financial incentive, which was to reduce the power consumption and replacement

cycle for individual luminaires. The other motivator was that the new system was de

signed to allow individual streetlights to be dimmed, thereby reducing light pollution. All

of the city’s nearly 20,000 streetlights were converted. It should be noted that other light

ing present in the grid cells was unaffected by the city’s LED conversion program. Fol

lowing the conversion, the majority of the new LED streetlights have been operated at

60% capacity from midnight to dawn [21].

Figure 13. Outline of the VIIRS DNB temporal profile grid constructed for Tucson, Arizona. The

Figure 13. Outline of the VIIRS DNB temporal profile grid constructed for Tucson, Arizona. The

locations of LED streetlight conversions are marked blue.

locations of LED streetlight conversions are marked blue.

For decades, Tucson has taken actions to reduce light pollution to limit interference

with several nearby astronomical observatories, such as Kitt Peak National Observatory. In 2016, the City of Tucson began replacing high-pressure sodium streetlights with LED

Remote Sens. 2022, 14, 1316 14 of 33

luminaires [21–23]. The motivation for the streetlight conversions was two-fold. There was

a financial incentive, which was to reduce the power consumption and replacement cycle

for individual luminaires. The other motivator was that the new system was designed

to allow individual streetlights to be dimmed, thereby reducing light pollution. All of

the city’s nearly 20,000 streetlights were converted. It should be noted that other lighting

present in the grid cells was unaffected by the city’s LED conversion program. Following

the conversion, the majority of the new LED streetlights have been operated at 60% capacity

from midnight to dawn [21].

The grid cells are divided into three zones to examine the effects of the streetlight

conversions on the index values. First, the grid cells are divided into those with and without

detectable lighting (Figure 14). Background areas, devoid of detectable lighting, are defined

based on annual average radiance values under one nanowatt/cm2/sr. The lit grid cells

are further divided into sets with and without City of Tucson LED streetlight conversions

Remote Sens. 2022, 14, x FOR PEER REVIEW 15 of 36  (Figure 15).

Remote Sens. 2022, 14, x FOR PEER REVIEW 15 of 36

Figure 14. Average radiance image from Tucson for the combined years 2015 and 2017. Background

Figure 14. Average radiance image from Tucson for the combined years 2015 and 2017. Background

areas, devoid of VIIRS detected lighting, are black. Grid cells with detectable lighting and no LED

areas, devoid of VIIRS detected lighting, are black. Grid cells with detectable lighting and no LED

streetlight conversions are gray, and lit grid cells with LED streetlight conversions are green.

Figure 14. Average radiance image from Tucson for the combined years 2015 and 2017. Background

streetlight conversions are gray, and lit grid cells with LED streetlight conversions are green.

areas, devoid of VIIRS detected lighting, are black. Grid cells with detectable lighting and no LED

streetlight conversions are gray, and lit grid cells with LED streetlight conversions are green.

Figure 15. The DNB temporal profiles are divided into three categories: background (blue), lit and

zero streetlight conversions (cyan), and lit with streetlight conversions (green). There are 1955 grid

cells in the background, 4121 grid cells lit with no-LED conversions, and 1124 grid cells with LED

Figure 15. The DNB temporal profiles are divided into three categories: background (blue), lit and

Figure 15. The DNB temporal profiles are divided into three categories: background (blue), lit and

streetlight conversions.

zero streetlight conversions (cyan), and lit with streetlight conversions (green). There are 1955 grid

zero streetlight conversions (cyan), and lit with streetlight conversions (green). There are 1955 grid

cells in the background, 4121 grid cells lit with no-LED conversions, and 1124 grid cells with LED

3. Results

streetlight conversions.

cells in the background, 4121 grid cells lit with no-LED conversions, and 1124 grid cells with LED

3.1. Re-Calculation of DNB Pixel Collection Times for HGS

streetlight conversions.

3. Results

The re-calculated SNPP DNB HGS pixel integration times are listed for each aggre

3.1. Re-Calculation of DNB Pixel Collection Times for HGS

gation zone in Table 2 and shown graphically in Figure 16. The integration time is longest

at nadir (2.68 ms) and shortest at the edge of scan (2.25 ms). The DNB pixel HGS-A and

The re-calculated SNPP DNB HGS pixel integration times are listed for each aggre

HGS-B integration times at nadir and edge of scan, shown overlaying a high-speed camera

gation zone in Table 2 and shown graphically in Figure 16. The integration time is longest

(1000 frames per second) temporal profile from a high-pressure sodium luminaire, are

at nadir (2.68 ms) and shortest at the edge of scan (2.25 ms). The DNB pixel HGS-A and

shown in Figure 17. The DNB HGS integration times are about one-third to one-fourth of

HGS-B integration times at nadir and edge of scan, shown overlaying a high-speed camera

the typical lighting flicker rates of 100 to 120 Hz.

(1000 frames per second) temporal profile from a high-pressure sodium luminaire, are

shown in Fiure 17. The DNB HGS interation times are about one-third to one-fourth of

Remote Sens. 2022, 14, 1316 15 of 33

A set of five Tucson grid cells were selected for more detailed examination and testing.

This included one grid cell from the background. The other four grid cells were selected

because they show abrupt and dramatic changes in radiance levels, with two from the

city’s LED conversion zone and two from areas outside the city’s LED conversion zone. For

these five grid cells, the annual percent flicker, flicker index, and index of dispersion values

were calculated.

3. Results

3.1. Re-Calculation of DNB Pixel Collection Times for HGS

The re-calculated SNPP DNB HGS pixel integration times are listed for each aggrega

tion zone in Table 2 and shown graphically in Figure 16. The integration time is longest

at nadir (2.68 ms) and shortest at the edge of scan (2.25 ms). The DNB pixel HGS-A and

Remote Sens. 2022, 14, x FOR PEER REVIEW 17 of 36  HGS-B integration times at nadir and edge of scan, shown overlaying a high-speed camera

(1000 frames per second) temporal profile from a high-pressure sodium luminaire, are

shown in Figure 17. The DNB HGS integration times are about one-third to one-fourth of

Remote Sens. 2022, 14, x FOR PEER REVIEW 17 of 36

the typical lighting flicker rates of 100 to 120 Hz.

Figure 16. SNPP DNB pixel integration times by aggregation zone.

Figure 16. SNPP DNB pixel integration times by aggregation zone.

Figure 16. SNPP DNB pixel integration times by aggregation zone.

Figure 17. SNPP VIIRS DNB pixel integration times for HGS-A and HGS-B randomly placed over a

high-pressure sodium temporal profile collected with the Sony high-speed camera. Each HGS sam

ple consists of an A and a B side, separated by a 0.25 ms gap.

Figure 17. SNPP VIIRS DNB pixel integration times for HGS-A and HGS-B randomly placed over a

Figure 17. SNPP VIIRS DNB pixel integration times for HGS-A and HGS-B randomly placed over a

3.2. Results from High-Speed Camera Temporal Profiles for Individual Sources

high-pressure sodium temporal profile collected with the Sony high-speed camera. Each HGS sam

high-pressure sodium temporal profile collected with the Sony high-speed camera. Each HGS sample

Figure 18 shows representative examples of the high-speed camera temporal profiles

ple consists of an A and a B side, separated by a 0.25 ms gap.

from the 23 individual luminaires, including high-pressure sodium, metal halide, fluores

consists of an A and a B side, separated by a 0.25 ms gap.

cent, incandescent, and LED. The 120 Hz flicker pattern can be seen visually, though the

3.2. Results from High-Speed Camera Temporal Profiles for Individual Sources

clarity of the pattern is diminished in the LED streetlight.

Figure 18 shows representative examples of the high-speed camera temporal profiles

from the 23 individual luminaires, including high-pressure sodium, metal halide, fluores

cent, incandescent, and LED. The 120 Hz flicker pattern can be seen visually, though the

clarity of the pattern is diminished in the LED streetlight.

Remote Sens. 2022, 14, 1316 16 of 33 Table 2. SNPP VIIRS DNB pixel integration times by aggregation zone.

Aggregation

Number of Sub-pixels per Pixel Number of

Time per

Time on

Time per

Total Time

Mode from Nadir

Track

Direction Scan Direction

Pixels per Mode

Pixel (s) HGS-A

HGS A-B Gap (s)

Pixel (s) HGS-B

per DNB Pixel (s)

1 42 66 184 1.21E-03 2.50E-04 1.21E-03 2.68E-03 2 42 64 72 1.20E-03 2.50E-04 1.20E-03 2.66E-03 3 41 62 88 1.20E-03 2.50E-04 1.20E-03 2.64E-03 4 40 59 72 1.19E-03 2.50E-04 1.19E-03 2.62E-03 5 39 55 80 1.17E-03 2.50E-04 1.17E-03 2.59E-03 6 38 52 72 1.16E-03 2.50E-04 1.16E-03 2.57E-03 7 37 49 64 1.15E-03 2.50E-04 1.15E-03 2.54E-03 8 36 46 64 1.14E-03 2.50E-04 1.14E-03 2.52E-03 9 35 43 64 1.12E-03 2.50E-04 1.12E-03 2.50E-03 10 34 40 64 1.11E-03 2.50E-04 1.11E-03 2.48E-03 11 33 38 64 1.11E-03 2.50E-04 1.11E-03 2.46E-03 12 32 35 80 1.09E-03 2.50E-04 1.09E-03 2.44E-03 13 31 33 56 1.09E-03 2.50E-04 1.09E-03 2.42E-03

14 30 30 80 1.07E-03 2.50E-04 1.07E-03 2.40E-03 15 29 28 72 1.07E-03 2.50E-04 1.07E-03 2.38E-03 16 28 26 72 1.06E-03 2.50E-04 1.06E-03 2.37E-03 17 27 24 72 1.05E-03 2.50E-04 1.05E-03 2.35E-03 18 27 23 32 1.05E-03 2.50E-04 1.05E-03 2.35E-03 19 26 22 48 1.04E-03 2.50E-04 1.04E-03 2.34E-03 20 26 21 32 1.04E-03 2.50E-04 1.04E-03 2.33E-03 21 25 20 48 1.04E-03 2.50E-04 1.04E-03 2.32E-03 22 25 19 40 1.03E-03 2.50E-04 1.03E-03 2.31E-03 23 24 18 56 1.03E-03 2.50E-04 1.03E-03 2.31E-03 24 24 17 40 1.02E-03 2.50E-04 1.02E-03 2.30E-03 25 23 16 72 1.02E-03 2.50E-04 1.02E-03 2.29E-03 26 23 15 24 1.02E-03 2.50E-04 1.02E-03 2.28E-03 27 22 15 32 1.02E-03 2.50E-04 1.02E-03 2.28E-03 28 22 14 64 1.01E-03 2.50E-04 1.01E-03 2.28E-03 29 21 13 64 1.01E-03 2.50E-04 1.01E-03 2.27E-03 30 21 12 64 1.01E-03 2.50E-04 1.01E-03 2.26E-03 31 20 12 16 1.01E-03 2.50E-04 1.01E-03 2.26E-03 32 20 11 80 1.00E-03 2.50E-04 1.00E-03 2.25E-03 Total 2032 7.71E-02

3.2. Results from High-Speed Camera Temporal Profiles for Individual Sources

Figure 18 shows representative examples of the high-speed camera temporal profiles

from the 23 individual luminaires, including high-pressure sodium, metal halide, fluores cent, incandescent, and LED. The 120 Hz flicker pattern can be seen visually, though the clarity of the pattern is diminished in the LED streetlight.

Remote Sens. 2022, 14, 1316 17 of 33 te Sens. 2022, 14, x FOR PEER REVIEW 19 of 36

Figure 18. High-speed camera temporal profiles from a variety of individual luminaires. The bright

Figure 18. High-speed camera temporal profiles from a variety of individual luminaires. The

ness axis scaling has been adjusted to highlight the flicker pattern, which is weakest in the LED. The

brightness axis scaling has been adjusted to highlight the flicker pattern, which is weakest in the LED.

title line on each profile records the luminaire type, sample number, percent flicker (PF), flicker

The title line on each profile records the luminaire type, sample number, percent flicker (PF), flicker

index (FI), and dispersion index (DI).

index (FI), and dispersion index (DI).

The temporal profiles were resampled to simulate DNB temporal profiles, a process

that obliterates the cycling pattern evident in the millisecond temporal profiles (Figure 19).

However, histograms from the DNB temporal profiles of individual luminaires continue

to exhibit spikes near the low and high ends of brightness range, corresponding to the

Remote Sens. 2022, 14, 1316 18 of 33

flicker inflection points. The histogram spike pattern is characteristic of “harmonic oscil

lators” [24]. The histogram spikes occur due to the increased temporal dwell at nearly

Remote Sens. 2022, 14, x FOR PEER REVIEW 20 of 36

the same brightness level when the sample falls across either the upper or lower flicker

inflection points.

Figure 19. Simulated DNB temporal profile and associated histogram from the resampling of a metal  Figure 19. Simulated DNB temporal profile and associated histogram from the resampling of a

halide high-speed camera temporal profile. Note that the brightness range of the simulated DNB  metal halide high-speed camera temporal profile. Note that the brightness range of the simulated

temporal profile is slightly less than the original 1000 frames per second temporal profile, an expres

DNB temporal profile is slightly less than the original 1000 frames per second temporal profile, an

sion of the temporal aggregation from the longer DNB pixel aggregation. The histogram shows  expression of the temporal aggregation from the longer DNB pixel aggregation. The histogram shows

spikes at brightness levels of 10.5 and 19, a vestigial expression of the flickering upper and lower  spikes at brightness levels of 10.5 and 19, a vestigial expression of the flickering upper and lower sine

sine wave peaks.

wave peaks.

The flicker index and ACF results from the individual luminaires are summarized

in Table 3. The table is sorted according to the percent flicker in descending order. The

highest percent flicker values are found with the high-intensity discharge lamps, with the

Remote Sens. 2022, 14, 1316 19 of 33

two high-pressure sodium sources having the highest percent flicker, followed by the metal

halides and low-pressure sodium. The LED streetlights and one of the fluorescent tubes

have the lowest percent flicker.

Table 3. Flicker results for individual luminaires. SDNB = simulated DNB.

Luminaires Number % Flicker

Compact

Flicker

Index Dispersion Flicker Hz

SDNB % Flicker

SDNB Flicker Index

SDNB

Dispersion

Fluorescent C0028 68.12 0.8216 17.1248 120 38.59 0.7303 8.0688 High Pressure

Sodium C0017 43.33 0.7250 9.3200 120 28.69 0.6356 5.4270 High Pressure

Sodium C0042 40.02 0.6398 9.3078 120 28.95 0.6249 5.6576 Metal halide C0012 21.71 0.5656 3.6492 120 18.49 0.5816 2.3106 Low Pressure

Sodium C0007 15.31 0.6992 1.0630 120 8.25 0.6051 0.4863 Metal Halide C0015 15.18 0.5627 2.0215 120 12.40 0.5550 1.3042 Ceramic metal

Halide C0009 14.40 0.5754 1.7200 120 10.73 0.6237 1.0180 Metal Halide RAB C0016 12.51 0.5956 1.7166 120 9.96 0.5474 0.8887 Incandescent C0027 5.47 0.5108 0.2264 60 5.26 0.5464 0.2031 Fluorescent C0006 1.70 0.5491 0.0137 120 1.23 0.5152 0.0080 Incandescent C0034 1.44 0.4794 0.0179 120 1.23 0.5604 0.0128 Incandescent dual C0033 1.30 0.4986 0.0135 120 1.14 0.5661 0.0093 Incandescent C0014 1.06 0.5379 0.0118 120 1.16 0.5075 0.0077 iphone flashlight C0036 0.77 0.4574 0.0047 0 0.53 0.6268 0.0016 Compact

Fluorescent C0031 0.65 0.5561 0.0031 120 0.47 0.6230 0.0018 Compact

Fluorescent C0032 0.63 0.3580 0.0028 120 0.59 0.5852 0.0013 LED C0018 0.29 0.5059 0.0007 120 0.30 0.5049 0.0005 LED 4000 K C0020 0.19 0.4897 0.0003 120 0.22 0.4930 0.0002 Compact

Fluorescent C0011 0.14 0.5450 0.0003 120 0.17 0.5378 0.0003 LED C0026 0.12 0.5250 0.0002 120 0.12 0.5201 0.0001 LED 3000 K C0022 0.10 0.4360 0.0001 120 0.11 0.5117 0.0001 Fluorescent

Ceiling C0035 0.08 0.5547 0.00004 120 0.09 0.6592 0.00004

Figure 20 shows representative examples of ACF profiles from the high-speed camera

data and a grid cell in Tucson found to have weekly brightness cycling. The ACF analysis found 120 Hz flicker in 19 of 20 individual luminaires (Table 3). The ACF analysis found no detectable flicker from the iPhone flashlight (sample C0036).

Remote Sens. 2022, 14, 1316 20 of 33 Remote Sens. 2022, 14, x FOR PEER REVIEW 21 of 36

Figure 20. Examples of autocorrelation function (ACF) profiles from the study. (A) High-speed cam

Figure 20. Examples of autocorrelation function (ACF) profiles from the study. (A) High-speed

era (HSC) data from a Sylvania incandescent light (C0027), (B) HSC data from an LED streetlight

camera (HSC) data from a Sylvania incandescent light (C0027), (B) HSC data from an LED streetlight

(C0018), (C) PMT-equipped HSC data from a lunar view, (D) HSC data from a lunar view, and (E)  (C0018), (C) PMT-equipped HSC data from a lunar view, (D) HSC data from a lunar view, and

DJI Air 2S drone camera data of lights in Moscow, Russia, collected at 120 frames per second. The  (E) DJI Air 2S drone camera data of lights in Moscow, Russia, collected at 120 frames per second. The

primary peak has a lag of six frames, indicating 20 Hz flickering. The actual flicker rate is 100 Hz.  primary peak has a lag of six frames, indicating 20 Hz flickering. The actual flicker rate is 100 Hz. The

The discrepancy is due to aliasing. (F) VIIRS DNB nightly temporal profile of Davis Monthan Air  discrepancy is due to aliasing. (F) VIIRS DNB nightly temporal profile of Davis Monthan Air Force

Force Base, Tucson, Arizona. The seven-day lag indicates the presence of weekly cycling in the  Base, Tucson, Arizona. The seven-day lag indicates the presence of weekly cycling in the brightness

brightness of the lights.

of the lights.

Scattergrams were generated to examine the relationships between the three indices

and the impact of DNB resampling. Figure 21 shows the percent flicker versus flicker index,

with labeling of the lighting types. The luminaires with high percent flicker also have

relatively high flicker index values, though it is not a well-defined linear relationship. In

contrast, the index of dispersion is highly correlated to percent flicker (Figure 22).

Remote Sens. 2022, 14, x FOR PEER REVIEW 23 of 36  Remote Sens. 2022, 14, 1316 21 of 33 Remote Sens. 2022, 14, x FOR PEER REVIEW 23 of 36

Figure 21. Percent flicker versus flicker index for individual luminaires, multi-light collections, and

Figure 21. Percent flicker versus flicker index for individual luminaires, multi-light collections, and

Figure 21. Percent flicker versus flicker index for individual luminaires, multi-light collections, and

Tucson DNB profiles. The individual luminaires are clustered into types: HPS = high-pressure so

Tucson DNB profiles. The individual luminaires are clustered into types: HPS = high-pressure so

Tucson DNB profiles. The individual luminaires are clustered into types: HPS = high-pressure sodium,

dium, MH = metal halide, LPS = low-pressure sodium, INC = incandescent, CF = compact fluores

dium, MH = metal halide, LPS = low-pressure sodium, INC = incandescent, CF = compact fluores

MH = metal halide, LPS = low-pressure sodium, INC = incandescent, CF = compact fluorescent, IP =

cent, IP = iPhone flashlight, LED = LED streetlights, FT = fluorescent tubes.

cent, IP = iPhone flashlight, LED = LED streetlights, FT = fluorescent tubes.

iPhone flashlight, LED = LED streetlights, FT = fluorescent tubes.

Figure 22. Percent flicker versus the index of dispersion for the HSC collections made of individual

Figure 22. Percent flicker versus the index of dispersion for the HSC collections made of individual

Figure 22. Percent flicker versus the index of dispersion for the HSC collections made of individual

luminaires, multi-light collections, and DNB temporal profiles from Tucson.

luminaires, multi-light collections, and DNB temporal profiles from Tucson.

luminaires, multi-light collections, and DNB temporal profiles from Tucson.

The scattergram of percent flicker from the individual luminaires versus DNB sim

ulated temporal profiles shows a highly linear relationship (Figure 23). There is a slight

decline in the percent flicker values from the DNB-simulated temporal profiles, presum

ably due to the longer integration time of the DNB versus the Sony camera running at

1000 frames per second. The flicker index values from HSC temporal profiles versus their

DNB-simulated equivalents are shown in Figure 24. Note that the DNB simulation results

in compression in the flicker index range. Figure 25 shows that the index of dispersion

values for HSC and DNB simulated temporal profiles are highly correlated.

Remote Sens. 2022, 14, x FOR PEER REVIEW 24 of 36  Remote Sens. 2022, 14, x FOR PEER REVIEW 24 of 36  Remote Sens. 2022, 14, 1316 22 of 33

Figure 23. Percent flicker from the high-speed camera collections versus the DNB-simulated tem

Figure 23. Percent flicker from the high-speed camera collections versus the DNB-simulated temporal

Figure 23. Percent flicker from the high-speed camera collections versus the DNB-simulated tem

poral profiles. The two are highly corelated, with the data cloud forming a diagonal. There is a slight

profiles. The two are highly corelated, with the data cloud forming a diagonal. There is a slight loss

poral profiles. The two are highly corelated, with the data cloud forming a diagonal. There is a slight

loss in flicker index values for the DNB-simulated sets, presumably due to the longer integration

in flicker index values for the DNB-simulated sets, presumably due to the longer integration time of

loss in flicker index values for the DNB-simulated sets, presumably due to the longer integration

time of the DNB compared to the digital camera data.

the DNB compared to the digital camera data.

time of the DNB compared to the digital camera data.

Figure 24. Flicker index values from the high-speed camera collections versus the DNB-simulated

Figure 24. Flicker index values from the high-speed camera collections versus the DNB-simulated

temporal profiles. The two data sets are not well correlated. Specifically, the flicker index data range

Figure 24. Flicker index values from the high-speed camera collections versus the DNB-simulated

temporal profiles. The two data sets are not well correlated. Specifically, the flicker index data range

temporal profiles. The two data sets are not well correlated. Specifically, the flicker index data range

from the simulated DNB temporal profiles is compressed, indicating a loss in the index’s ability to

from the simulated DNB temporal profiles is compressed, indicating a loss in the index’s ability to

from the simulated DNB temporal profiles is compressed, indicating a loss in the index’s ability to

distinguish different types of lighting.

distinguish different types of lighting.

distinguish different types of lighting.

Remote Sens. 2022, 14, x FOR PEER REVIEW 25 of 36

Remote Sens. 2022, 14, x FOR PEER REVIEW 25 of 36  Remote Sens. 2022, 14, 1316 23 of 33

Figure 25. Index of dispersion values from the high-speed camera collections versus the DNB sim

ulated temporal profiles. The two are highly corelated, with the data cloud forming a diagonal. The

Figure 25. Index of dispersion values from the high-speed camera collections versus the DNB

DNB-simulated profiles produced slightly lower dispersion values, presumably due to the longer

Figure 25. Index of dispersion values from the high-speed camera collections versus the DNB sim

simulated temporal profiles. The two are highly corelated, with the data cloud forming a diagonal.

integration time of the DNB compared to the digital camera data.

ulated temporal profiles. The two are highly corelated, with the data cloud forming a diagonal. The

The DNB-simulated profiles produced slightly lower dispersion values, presumably due to the longer

DNB-simulated profiles produced slightly lower dispersion values, presumably due to the longer

3.3. Results from Lunar Views

integration time of the DNB compared to the digital camera data.

integration time of the DNB compared to the digital camera data.

The lunar view temporal profiles from the Sony and PMT cameras are shown in Fig

3.3. Results from Lunar Views

3.3. Results from Lunar Views

ure 26. These can be considered HSC collections of individual sources since all other

The lunar view temporal profiles from the Sony and PMT cameras are shown in

The lunar view temporal profiles from the Sony and PMT cameras are shown in Fig

sources are masked out. The temporal profiles from the Sony camera show a low level of

Figure 26. These can be considered HSC collections of individual sources since all other

ure 26. These can be considered HSC collections of individual sources since all other

background noise with no sign of flickering. The PMT’s lunar profile shows prominent

sources are masked out. The temporal profiles from the Sony camera show a low level of

sources are masked out. The temporal profiles from the Sony camera show a low level of

AC fluctuations. This led to the exclusion of the PMT collections from other parts of this

background noise with no sign of flickering. The PMT’s lunar profile shows prominent AC

background noise with no sign of flickering. The PMT’s lunar profile shows prominent

study.

fluctuations. This led to the exclusion of the PMT collections from other parts of this study.

AC fluctuations. This led to the exclusion of the PMT collections from other parts of this

study.

Figure 26. High-speed camera collections of the moon at night from the Sony camera (top) and the

Figure 26. High-speed camera collections of the moon at night from the Sony camera (top) and

Sony camera equipped with a photo-multiplier tube (bottom). Note the prominent electronic flicker

the Sony camera equipped with a photo-multiplier tube (bottom). Note the prominent electronic

present in the PMT collection. This led to the exclusion of the PMT from further flicker study collec

Figure 26. High-speed camera collections of the moon at night from the Sony camera (top) and the

flicker present in the PMT collection. This led to the exclusion of the PMT from further flicker study

tions.

Sony camera equipped with a photo-multiplier tube (bottom). Note the prominent electronic flicker

collections.

present in the PMT collection. This led to the exclusion of the PMT from further flicker study collec

tions.

Remote Sens. 2022, 14, 1316 24 of 33

3.4. Results from High-Speed Camera Temporal Profiles for Multi-Light Collections

Figure 27 shows the high-speed camera temporal profiles from the multi-light col

lections from the table-top collection, oblique view of Golden, horizontal view of the

Broomfield Civic Center, panoramic view of Boulder, and overhead view of outdoor light ing collected from a hovering drone in Moscow. Flicker is evident in each of the profiles,

though it is more weakly expressed in the Golden and Boulder samples where the sample

view covered lights present in multiple VIIRS DNB pixel footprints. The multi-light tempo ral profiles from Colorado feature 120 Hz flicker. Note that the horizontal axis scaling is

expanded from 100 ms to 500 ms for the Moscow drone profile where the camera frame

rate is 120 per second. The index results are reported in Table 4. All five of the multi-light

temporal profiles had flicker present in the range of 3 to 5%. The ACF analysis found the

Remote Sens. 2022, 14, x FOR PEER REVIEW 27 of 36  flicker rate from Colorado to be 120 Hz, and the Moscow sample’s to be 100 Hz.

Figure 27. High-speed camera temporal profiles from five multi-light collections. Each of the samples

Figure 27. High-speed camera temporal profiles from five multi-light collections. Each of the sam

ples shows visual evidence of AC flicker. The pattern is less clear in the Golden and Boulder exam

shows visual evidence of AC flicker. The pattern is less clear in the Golden and Boulder examples,

ples, where the field-of-view covered multiple DNB pixels. Note that the temporal scale for the Col

where the field-of-view covered multiple DNB pixels. Note that the temporal scale for the Colorado

orado 1000 frames per second collection is standardized to 100 ms. The Moscow drone data were

1000 frames per second collection is standardized to 100 ms. The Moscow drone data were collected

collected at 120 frames per second and a horizontal axis range of 500 ms to expose the flicker pattern.

at 120 frames per second and a horizontal axis range of 500 ms to expose the flicker pattern. PF =

PF = percent flicker, FI = flicker index, DI = dispersion index.

percent flicker, FI = flicker index, DI = dispersion index.

3.5. Examination of DNB Temporal Profiles of Tucson, Arizona

The VIIRS DNB profiles for five grid cells from the Tucson metropolitan area are

shown in Figure 28. Percent flicker, flicker index, and index of dispersion calculations

were made for each of the five temporal profiles to examine the value of the indices in

detecting changes in flicker. For each site, one year is reported as a reference and a second

year is reported as a subject. The results are shown in Table 5.

Remote Sens. 2022, 14, 1316 25 of 33 Table 4. Multi-light camera collections. SDNB = simulated VIIRS DNB.

Observed

Scene Number % Flicker Flicker

Index Dispersion Flicker Hz DNB %

Flicker

Table Mul

DNB

Flicker Index

DNB

Dispersion

tilights C0005 8.38 0.5277 0.1119 120 6.67 0.4953 0.072

Golden

Panorama C0040 8.43 0.5178 0.1972 120 9.92 0.5069 0.1581 Broomfield

Civic

Center Boulder

C0146 4.79 0.5689 0.0991 120 4.36 0.5214 0.06

Panorama C0138 5.09 0.5274 0.0585 120 4.99 0.4956 0.0349

Moscow

Drone D0014 3.61 0.5008 0.0228 100 2.45 0.5026 0.0101

3.5. Examination of DNB Temporal Profiles of Tucson, Arizona

The VIIRS DNB profiles for five grid cells from the Tucson metropolitan area are

shown in Figure 28. Percent flicker, flicker index, and index of dispersion calculations were made for each of the five temporal profiles to examine the value of the indices in detecting changes in flicker. For each site, one year is reported as a reference and a second year is reported as a subject. The results are shown in Table 5.

Table 5. Flicker index calculations for five Tucson grid cells.

Location Cell Number

S. Kino Parkway

Reference Year

Ref. % Flicker

Ref.

Flicker Index

Ref.

Dispersion

2017 % Flicker

2017

Flicker Index

2017

Dispersion

LED Con versions

and 22nd 4738 2015 21.21 0.5797 1.88 23.29 0.5148 0.91 61 Automall 2730 2015 63.77 0.677 13.92 59.09 0.6725 9.01 29 HomeGoods

Distribution Center

6746 2014 99.99 0.6175 4.15 41.63 0.5644 10.87 0

Airport terminal 6841 2013 22.59 0.657 15.17 36.06 0.6457 7.13 0 Mt. Lemmon 100 2015 99.97 0.6426 0.422 99.96 0.6554 0.404 0

The South Kino Parkway grid cell covers a lit roadway and a commercial zone with

streets and buildings. It had 61 LED streetlight conversions. The temporal profile started out in 2012 with a mean radiance of 46 nW and a variance of 141, more than three times the mean. There was an upward radiance trend from 2012 to 2015, leading to a 2015 mean of 62 nW and a variance of 117. In 2016, there was a rapid decline in both radiance and variance, likely in response to the reduction in streetlight voltages after midnight and the near absence of flicker in the LED streetlights. The radiant emission pattern remained stable from 2017 to 2021.

The Tucson Automall grid cell is centered on a large, brightly lit parking lot featuring

a large number of metal halide and high-pressure sodium luminaires. Multiple car dealer ships are located together and are reported to begin turning off portions of their lighting after the 8 p.m. showroom closures. The grid cell had 29 LED streetlight conversions. The temporal profile is distinctive relative to the others in Figure 28 for its high level of variance across all years, fluctuating from a high of 1796 in 2013 to a low of 568 in 2018. As with the Kino Parkway grid cell, there is a decline in the mean radiance in 2016 during the

Remote Sens. 2022, 14, 1316 26 of 33

Remote Sens. 2022, 14, x FOR PEER REVIEW 28 of 36  LED conversion period. The mean radiance drops from 101 nW in 2015 to 72 nW in 2018.

Following this, the mean radiance then slowly rebounds to 77 nW in 2021.

Figure 28. VIIRS DNB temporal profiles for five grid cells from Tucson, Arizona.

Figure 28. VIIRS DNB temporal profiles for five grid cells from Tucson, Arizona.

The Tucson International Airport (TUS) terminal grid cell contains the terminal build

ing, parking lots, and a portion of the runway. It is one of the brightest locations found

in Tucson, with zero LED streetlight conversions. The mean brightness started out with a

Remote Sens. 2022, 14, 1316 27 of 33

mean of 220 nW and variance of 3636 in 2012. The mean increased slightly until mid-2014,

at which point there was a sharp dip in radiance which continued until mid-2015, a period

of renovation with the conversion of many metal halide and high-pressure sodium outdoor

lights to highly-shielded LEDs. Once the construction was complete, there was a slight

radiance recovery in late 2015, and the emissions then stabilized to means near 120 nW and

variances near 900 from 2017 to 2021.

The HomeGoods Distribution Center is an 850,000 square foot facility that held its

grand opening on 14 October 2016 [25]. The DNB temporal profile shows a relatively

modest and stable level of lighting present from 2012 to July 2015, with mean radiances

near 10 nW and variances ranging from 35 to 50. The brightness dropped slightly in August

of 2015 and then bumped up with the radiances frequently rising to the 20–30 nW range

for the next year—the facilities construction phase. The brightness increased again at the

end of June 2016, prior to the grand opening, and remained in a largely stable state, with

mean radiances in the 80 to 90 nW range and variances in the 880 to 1000 range through

2021. There was a month-long radiance drop in April 2020, likely the result of a site-wide

shut down during the COVID-19 pandemic.

Remote Sens. 2022, 14, x FOR PEER REVIEW 30 of 36  The fifth individual grid cell being examined is representative of the unlit background,

taken from the core of Mt. Lemmon, part of the Coronado National Forest. The DNB

temporal profile features low mean radiances, in the range of 0.3 to 0.5 nW, indicating the

years. There was a series of radiance spikes, up to 8 nW on a winter night with high lunar  absence of detectable lighting. Variances were also low, under 1 in each of the years. There

illuminance, likely the result of moonlight reflecting off snow.

was a series of radiance spikes, up to 8 nW on a winter night with high lunar illuminance,

likely the result of moonlight reflecting off snow.

3.6. Examination of Annual Mean, Variance, and Dispersion across Tucson

The grid cells from the Tucson DNB temporal profile grid were divided into three

3.6. Examination of Annual Mean, Variance, and Dispersion across Tucson

zones: background (no lighting detected), lit and no-LED conversions, and lit with LED  The grid cells from the Tucson DNB temporal profile grid were divided into three

conversions. In this section, we examine the behavior of the lighting in the three zones  zones: background (no lighting detected), lit and no-LED conversions, and lit with LED

prior to and following the completion of most of the LED conversions using the annual

conversions. In this section, we examine the behavior of the lighting in the three zones

mean, variance, and index of dispersion from 2015 and 2017.

prior to and following the completion of most of the LED conversions using the annual

Figure 29 shows the scattergrams of the annual mean radiances for the three zones.

mean, variance, and index of dispersion from 2015 and 2017.

There was a slight increase in the mean radiance for 71.6% of the grid cells in the back

Figure 29 shows the scattergrams of the annual mean radiances for the three zones.

ground zone. The mean radiances were largely unchanged in the lit grid cells with no LED

There was a slight increase in the mean radiance for 71.6% of the grid cells in the background

streetlight conversions. In contrast, the mean radiance declined for 97% of the grid cells

zone. The mean radiances were largely unchanged in the lit grid cells with no LED

with LED conversion from 2015 to 2017.

streetlight conversions. In contrast, the mean radiance declined for 97% of the grid cells

with LED conversion from 2015 to 2017.

Figure 29. Mean radiance scattergrams for the three zones for 2015 versus 2017. The diagonal lines

Figure 29. Mean radiance scattergrams for the three zones for 2015 versus 2017. The diagonal lines  indicate equal values in the two years. There was a minor increase in the mean radiance in 2017 for

indicate equal values in the two years. There was a minor increase in the mean radiance in 2017 for  the background grid cells. The mean radiance stayed nearly constant for lit grid cells lacking LED

the background grid cells. The mean radiance stayed nearly constant for lit grid cells lacking LED  streetlight conversions, tracking the diagonal. Mean radiance declined in 97% of the grid cells with

streetlight conversions, tracking the diagonal. Mean radiance declined in 97% of the grid cells with  LED streetlight conversions. Red triangles correspond to the five Tucson DNB profiles shown in

LED streetlight conversions. Red triangles correspond to the five Tucson DNB profiles shown in

Figure 28: ML = Mount Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon Interna

Figure 28: ML = Mount Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International

tional Airport; K = South Kino Parkway; TA = Tucson Automall.

Airport; K = South Kino Parkway; TA = Tucson Automall.

Figure 30 shows the scattergrams of the annual variance for the three zones for 2015

versus 2017. The grid cells in the background zone changed very little in variance from  2015 to 2017, with the data cluster centered on the diagonal. There was a discernable de

cline in variance for a small portion of the background grid cells. For the lit zone with no

LED conversions, 74.6% of the lit grid cells had a decline in variance. Amongst the grid

Remote Sens. 2022, 14, 1316 28 of 33

Figure 30 shows the scattergrams of the annual variance for the three zones for 2015

versus 2017. The grid cells in the background zone changed very little in variance from 2015

to 2017, with the data cluster centered on the diagonal. There was a discernable decline

Remote Sens. 2022, 14, x FOR PEER REVIEW 31 of 36

Remote Sens. 2022, 14, x FOR PEER REVIEW 31 of 36  in variance for a small portion of the background grid cells. For the lit zone with no-LED

conversions, 74.6% of the lit grid cells had a decline in variance. Amongst the grid cells

with LED streetlight conversions, 92.6% had a decline in variance in 2017 relative to 2015.

Figure 30. Variance scattergrams from the three zones for 2015 versus 2017. The diagonal lines

Figure 30. Variance scattergrams from the three zones for 2015 versus 2017. The diagonal lines in

Figure 30. Variance scattergrams from the three zones for 2015 versus 2017. The diagonal lines in

indicate equal values in the two years. The grid cells in the background zone were largely stable in

dicate equal values in the two years. The grid cells in the background zone were largely stable in

dicate equal values in the two years. The grid cells in the background zone were largely stable in

variance, with only a small portion of the grid cells dropping slightly in variance in 2017 relative to

variance, with only a small portion of the grid cells dropping slightly in variance in 2017 relative to

variance, with only a small portion of the grid cells dropping slightly in variance in 2017 relative to

2015. For the lit grid cells with no LED streetlight conversion, 74.6% had a slight decline in variance

2015. For the lit grid cells with no LED streetlight conversion, 74.6% had a slight decline in variance

2015. For the lit grid cells with no LED streetlight conversion, 74.6% had a slight decline in variance

in 2017. In contrast, 92.6% of the grid cells with LED streetlight conversions had a decline in variance

in 2017. In contrast, 92.6% of the grid cells with LED streetlight conversions had a decline in variance

in 2017. In contrast, 92.6% of the grid cells with LED streetlight conversions had a decline in variance

in 2017. Red triangles correspond to the five Tucson DNB profiles shown in Figure 28: ML = Mount

in 2017. Red triangles correspond to the five Tucson DNB profiles shown in Figure 28: ML = Mount

in 2017. Red triangles correspond to the five Tucson DNB profiles shown in Figure 28: ML = Mount

Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International Airport; K = South

Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International Airport; K = South

Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International Airport; K = South

Kino Parkway; TA = Tucson Automall.

Kino Parkway; TA = Tucson Automall.

Kino Parkway; TA = Tucson Automall.

Figure 31 shows the scattergrams of the annual index of dispersion for the three zones

Figure 31 shows the scattergrams of the annual index of dispersion for the three zones

Figure 31 shows the scattergrams of the annual index of dispersion for the three zones

for 2015 versus 2017. There was a slight decrease in the dispersion for the background

for 2015 versus 2017. There was a slight decrease in the dispersion for the background

for 2015 versus 2017. There was a slight decrease in the dispersion for the background zone.

zone. The dispersion is largely unchanged in the lit zone with no LED streetlight conver

zone. The dispersion is largely unchanged in the lit zone with no LED streetlight conver

The dispersion is largely unchanged in the lit zone with no LED streetlight conversions.

sions. There is a decline in the dispersion in 2017 compared to 2015 for 79.3% of the grid

sions. There is a decline in the dispersion in 2017 compared to 2015 for 79.3% of the grid

There is a decline in the dispersion in 2017 compared to 2015 for 79.3% of the grid cells

cells with LED streetlight conversions.

cells with LED streetlight conversions.

with LED streetlight conversions.

TUS

TUS

TA

TA

HG

HG

K

K

ML

ML

Figure 31. Index of dispersion scattergrams for the three zones for 2015 versus 2017. The diagonal Figure 31. Index of dispersion scattergrams for the three zones for 2015 versus 2017. The diagonal

Figure 31. Index of dispersion scattergrams for the three zones for 2015 versus 2017. The diagonal  lines indicate equal values in the two years. The grid cells in the background zone were largely

lines indicate equal values in the two years. The grid cells in the background zone were largely

lines indicate equal values in the two years. The grid cells in the background zone were largely stable stable in dispersion, with only a small portion of the grid cells dropping slightly in variance in 2017

stable in dispersion, with only a small portion of the grid cells dropping slightly in variance in 2017

in dispersion, with only a small portion of the grid cells dropping slightly in variance in 2017 relative relative to 2015. For the lit grid cells with no LED streetlight conversion, 79.3% had a decline in

relative to 2015. For the lit grid cells with no LED streetlight conversion, 79.3% had a decline in

to 2015. For the lit grid cells with no LED streetlight conversion, 74.8% had a decline in dispersion in dispersion in 2017. In contrast, 79.3% of the grid cells with LED streetlight conversions had a decline

dispersion in 2017. In contrast, 79.3% of the grid cells with LED streetlight conversions had a decline  2017. In contrast, 79.3% of the grid cells with LED streetlight conversions had a decline in dispersion in dispersion in 2017. Red triangles correspond to the five Tucson DNB profiles shown in Figure 28:

in dispersion in 2017. Red triangles correspond to the five Tucson DNB profiles shown in Figure 28:  in 2017. Red triangles correspond to the five Tucson DNB profiles shown in Figure 28: ML = Mount ML = Mount Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International Airport;

ML = Mount Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International Airport;  Lemmon; HG = HomeGoods Distribution Center; TUS = Tuscon International Airport; K = South K = South Kino Parkway; TA = Tucson Automall.

K = South Kino Parkway; TA = Tucson Automall.

Kino Parkway; TA = Tucson Automall.

4. Discussion

4. Discussion

The recalculation of the VIIRS DNB HGS pixel aggregation time finds that the pixel

The recalculation of the VIIRS DNB HGS pixel aggregation time finds that the pixel  collection times vary from 2.25 to 2.68 ms. This is substantially shorter than the 8.3 ms

collection times vary from 2.25 to 2.68 ms. This is substantially shorter than the 8.3 ms  cycle for the lighting flicker associated with a 60 Hz alternating current. Of the lighting

cycle for the lighting flicker associated with a 60 Hz alternating current. Of the lighting  types measured, flicker is most prominent in high-intensity discharge (HID) luminaires

types measured, flicker is most prominent in high-intensity discharge (HID) luminaires  such as high-pressure sodium and metal halide, which are widely used in outdoor light

such as high-pressure sodium and metal halide, which are widely used in outdoor light

Remote Sens. 2022, 14, 1316 29 of 33

4. Discussion

The recalculation of the VIIRS DNB HGS pixel aggregation time finds that the pixel

collection times vary from 2.25 to 2.68 ms. This is substantially shorter than the 8.3 ms cycle

for the lighting flicker associated with a 60 Hz alternating current. Of the lighting types

measured, flicker is most prominent in high-intensity discharge (HID) luminaires such as

high-pressure sodium and metal halide, which are widely used in outdoor lighting. Com pared to HID sources, flicker is reduced in incandescent and fluorescent lights. Fluorescent

light fixtures frequently include capacitors, which reduce the voltage fluctuations that cause

flicker. The thermal inertia of incandescent filaments serves as an inbuilt capacitor, reducing

flicker to a greater degree than HID sources. Over the years, LED manufacturers have

invested in the development of “constant-current” drivers [26] that have largely eliminated

flicker in today’s LED streetlights.

Evidence for the effects of flicker can be seen in VIIRS DNB temporal profiles from

Tucson sites where LED conversion projects replaced HID streetlights with low-flicker LED

sources. Both the South Kino Parkway and Tucson Airport temporal profiles start out with

high radiance levels and high variance. The conversion period is indicated by noticeable

drops in radiance levels. Upon completion of the LED conversions, the radiance levels

stabilize at a reduced level and both variance and dispersion are reduced. The radiance

drop is likely the result of voltage reductions after midnight, plus the improved shielding

of the new LED streetlights. In the grid cells where the variance reductions exceed that

radiance reduction, there is a decline in dispersion. This occurred in 79.3% of the Tucson

grid cells with LED streetlight conversions. This decline in the dispersion indicates that the

flicker was reduced when the high-flicker HID lights were replaced with low-flicker LED

lights.

Three indices were tested for their ability to detect changes in the flicker levels in VIIRS

DNB temporal profiles, including percent flicker, flicker index, and the index of dispersion. Percent flicker and the flicker index are standard measurements developed to detect and

quantify flicker by the lighting engineering community. All three indices performed well

for the individual luminaires listed in Table 3. For the VIIRS DNB profiles, the index testing

contrasted the values from a reference year and a subject year, selected to straddle an

obvious change in the lighting behavior. The DNB profile index results (Table 4) indicate

that the percent flicker and flicker index did not perform as well as the index of dispersion. The grid cell from Figure 16 with the highest percent flicker is from Mt. Lemmon—a

background area devoid of lighting. The percent flicker remained nearly constant for the

South Kino Parkway grid cell, which had 61 LED streetlight conversions, resulting in a

substantial reduction in variance. Similar issues were found for the flicker index, which

jumped by two orders of magnitude for the Mt. Lemmon grid cell and remained largely

unchanged for the South Kino Parkway and Tucson Airport grid cells, which had large

numbers of HID-to-LED conversions. Our assessment is that the index of dispersion is

better than the percent flicker and flicker index for the detection and analysis of flicker in

VIIRS DNB temporal profiles.

5. Conclusions

Back to the original question, is the VIIRS DNB a flicker meter in space? The answer is

“yes” and “no”. The DNB HGS pixel integration time is more than three times shorter than

the 100 to 120 Hz flicker found in lighting. However, VIIRS is incapable of measuring the

flicker rates in Hz due to the fact that the DNB’s repeat cycle is measured in hours rather

than milliseconds. The VIIRS DNB differs from conventional flicker meters in that it is

unable to collect coherent flicker cycles from spaces. Instead, the DNB integrates radiances

from all of the surface lighting present in pixel footprints and collects for 2–3 ms nightly

with hour- to day-long gaps between observations. This implies that if flicker is present at

the pixel footprint level, the recorded VIIRS radiance will fluctuate unpredictably over time,

making it a source of instantaneous dispersion. This instability is expressed as an enhanced

level of variance in long-term DNB temporal profiles. Because the VIIRS pixel footprints are

Remote Sens. 2022, 14, 1316 30 of 33

about half a square kilometer, there can be many individual lights present and contributing

to the radiance observed by the DNB. If the flickering lights are not synchronized, the

observable radiance pattern is an erratic cacophony, with multiple offset flicker patterns

tending to cancel each other out. An AC flicker pattern can emerge from the cacophony,

though it will be a weak signal compared to that found in individual sources.

Several recent studies [15,27] have enumerated the factors that introduce variance into

VIIRS DNB temporal profiles, such as the view angle, atmospheric variability, cloud-effects, cycling of lunar illuminance, snow cover, and even deciduous tree phenology. To this,

we should add flicker and the variability of flicker within DNB pixel footprints that are

assembled in the construction of DNB temporal profiles. The distinction between flicker

and these other variables is that flicker is an instantaneous source of dispersion, and the

other sources operate with longer time constants. Together, these variables produce a cloud

of radiance dispersion in long-term DNB temporal profiles. The stability of DNB temporal

profiles can be improved by adjusting the radiance levels for individual pixels designed

to remove these influences. The purpose of cloud-screening is to reduce the radiance

dispersion induced by cloud obscuration, a variability that is unrelated to actual surface

lighting. Satellite zenith angle effects can be analyzed and removed via normalization [15]. It is also possible to isolate and remove fluctuations associated with lunar, weekly and

annual cycling from DNB temporal profiles. However, no amount of adjustment for the

environmental and geometric effects on DNB radiances can remove the effects of lighting

flicker. Thus, flicker, if present at the pixel level, imprints itself indelibly on the DNB

temporal profiles and sets an implicit limitation on the uncertainty of applications that

utilize the DNB observations of artificial lights.

It is possible to collect data on flicker levels with high-speed cameras. A frame rate

of 1000 per second makes it possible to observe flicker patterns in good detail. It appears

feasible to collect quantitative flicker information for footprints approximating the DNB

with high-speed cameras from hovering drones. One caution is that it is important to test

the camera for the presence of electronic flicker by viewing the moon or daylit surface

devoid of AC flicker.

The data from a high-speed camera collected on a wide range of luminaires indicate

that high-intensity discharge (HID) luminaries, such as high- and low-pressure sodium

and metal halides, are particularly prone to high levels of flicker. In contrast, modern LED

luminaires are virtually devoid of flicker. This fact should be considered when designing

ground-based DNB calibration sources. LED sources are vastly better than HID sources

for DNB calibration sources. This fact casts doubt on the value of DNB calibration efforts

where HID sources are utilized [28,29].

We examined the DNB temporal profiles from grid cells known to have had HID-to

LED conversions in Tucson, Arizona. The DNB profiles reveal the conversion date range,

and the reduction in flicker effects that occurs when temporally unstable HID luminaires

are replaced with steady LED sources. Three annual indices were tested for detecting and

characterizing flicker in DNB temporal profiles, including percent flicker, flicker index, and

the index of dispersion. The preliminary results indicate that the index of dispersion is

the best of the three. The flicker index values calculated from the DNB temporal profiles

have a compressed range and show little change between the pre- and post-LED streetlight

conversion radiance profiles. Percent flicker operates suitably in lit grid cells but produces

anomalously high values for background grid cells.

It should be noted that the flicker effects on VIIRS DNB data cannot be detected or

analyzed using the current VIIRS monthly or annual nighttime light products, which only

report the average cloud-free radiance [30,31]. The analyses from Tucson were based on

nightly DNB temporal profiles. Our results indicate that the information value of monthly

and annual VIIRS DNB products can be improved by producing the full suite of statistical

moments (mean, variance, skew, and kurtosis). The mean and variance can be further

processed to calculate the index of dispersion.

Remote Sens. 2022, 14, 1316 31 of 33

Over time, outdoor lighting flicker is being reduced as high-pressure sodium and metal

halide sources are replaced with constant-current LED streetlights [32]. The results from

Tucson indicate that the loss of flicker and dimming of the upwelling radiances associated

with LED conversions can be detected and quantified with VIIRS DNB temporal profiles. While the transition to LED is primarily based on economics, there may be human health

impacts. Several studies have noted the adverse health impacts of white LEDs, where the

blue component is enhanced [33,34]. In contrast, the reduction in flicker associated with

LED conversions may have beneficial effects on humans, a possibility that should be further

investigated.

Our results shed some light on the sensor requirements for the observation of flicker

rates and amplitudes. Flicker can be detected and analyzed with high-speed camera data. Such data can be collected on the ground or from the air. The collection platform should

be stabilized with a tripod on the ground or by hovering in the air. Airborne surveys of

flicker will be the easiest way to investigate the spatial patterns of flicker synchronization

associated with multiple lights operating from a common AC power supply. It may be

possible to map boundaries between power grid entities based on the discontinuities in

flickering. From satellites, it may be possible to observe the flicker rates and amplitudes

with high-speed, low-detection-limit cameras capable of staring at lights present on the

earth’s surface. This could be done from geostationary orbit or with pointable sensors in

lower orbit.

In conclusion, VIIRS DNB-measured radiances from artificial light sources are affected

by several phenomena, including flicker. The flicker effects can be concentrated by filtering

out the pixels contaminated by cloud cover and adjusting the DNB radiances for effects

such as lunar reflectance, view angle, and atmospheric variability. In addition, outlier and

trend removal may be required to achieve consistent results. After the extraneous effects

are removed or compensated for, DNB temporal profiles can reveal details of lighting

development and conversions from high-flicker to low-flicker sources.

Author Contributions: Conceptualization, data collection, analysis, figure preparation, and writing: C.D.E.; data collection and analysis: M.Z.; data collection and provision of luminaires: D.K.; data

collection, manuscript review, and editing: S.D.M., S.J.A., P.C.S. and D.H.; data collection and

DNB temporal profile generation: F.C.H.; details on Tucson study sites: J.B., C.K.M. and C.C.M.K.;

conceptualization and administrative support: M.B.; conceptualization: J.T.; manuscript review

and editing: J.B., W.S.K., D.W.P., C.C.M.K., T.G. and D.H. All authors have read and agreed to the

published version of the manuscript.

Funding: This research was funded by the Rockefeller Foundation, grant number 2018 POW 004 (Prof. Jay Taneja, principal investigator) and the NOAA Joint Polar Satellite System (JPSS) proving ground

program project on VIIRS detection of COVID impacts on lighting (Prof. Steve Miller, principal

investigator). Dr. Kyba is funded by Helmholtz Association Initiative and Networking Fund (CS-003).

Data Availability Statement: The VIIRS DNB temporal profile grid of Tucson is available at: https:

//eogdata.mines.edu/wwwdata/hidden/dnb_profiles_deliver_licorr/usa_tucson2/ (accessed on 6

March 2022).

Acknowledgments: The VIIRS data used in the research are the result of many years of develop ment and satellite operations sponsored jointly by NASA and NOAA. Funding for the study came

from NOAA’s Joint Polar Satellite System (JPSS) proving ground study on COVID effects on light

ing (Steven Miller, principal investigator) and the Rockefeller Foundation’s e-guide project, grant

2018 POW 004 (Jay Taneja, principal investigator). Jeff Puschell of Raytheon Corporation assisted

in clarifying the DNB pixel aggregation timeline. We are thankful to Eng. Artem Khuramshin

(Hewlett Packard, Russia) for providing and operation of the drone during the slow-motion filming

of streetlights in Moscow.

Conflicts of Interest: The authors declare no conflict of interest.

Remote Sens. 2022, 14, 1316 32 of 33

References

1. Mirra, C. Flicker Measurement and Evaluation; Union Internationale d’Electrothermie, Group de Travail Perturbations: Paris, France, 1986.

2. Fan, Y.H.; Wu, C.J.; Fan, C.C.; Chih, K.W.; Liao, L.D. A Novel Time Based Current Compensator for LED Back Light Modules. Key Eng. Mater. 2007, 364–366, 377–382. [CrossRef]

3. Bartley, S.H. The neural determination of critical flicker frequency. J. Exp. Psychol. 1937, 21, 678. [CrossRef] 4. Wilkins, A.; Veitch, J.; Lehman, B. LED lighting flicker and potential health concerns: IEEE standard PAR1789 update. In Proceedings of the 2010 IEEE Energy Conversion Congress and Exposition, ECCE, Atlanta, GA, USA, 12–16 September 2010; pp. 171–178. [CrossRef]

5. Wilkins, A.J. Health and efficiency in lighting practice. Energy 1993, 18, 123–129. [CrossRef]

6. Price, L.L.A. Can the Adverse Health Effects of Flicker from LEDs and Other Artificial Lighting Be Prevented? LEUKOS 2017, 13, 191–200. [CrossRef]

7. Davis, J.; Hsieh, Y.-H.; Lee, H.-C. Humans perceive flicker artifacts at 500 Hz. Sci. Rep. 2015, 5, 7861. [CrossRef] 8. IEEE Recommended Practices for Modulating Current in High-Brightness LEDs for Mitigating Health Risks to Viewers. IEEE-SA Standards Board, 26 March 2015. Available online: http://www.bio-licht.org/02_resources/info_ieee_2015_standards-1789.pdf (accessed on 5 March 2022).

9. Takahashi, T.; Tsukahara, Y.; Nomura, M.; Matsuko, H. Pokemon Seizures. Neurol. J. Southeast Asia 1999, 4, 1–11. Available online: http://www.neurology-asia.org/articles/19991_001.pdf (accessed on 5 March 2022).

10. Inger, R.; Bennie, J.; Davies, T.W.; Gaston, K.J. Potential biological and ecological effects of flickering artificial light. PLoS ONE 2014, 9, e98631. [CrossRef]

11. Schakel, M.; Banerjee, K.; Bergen, T.; Blattner, P.; Bouroussis, C.; Dekker, P.; Klej, A.; Li, C.; Ootake, H.; Reiners, T.; et al. Guidance on the Measurement of Temporal Light Modulation of Light Sources and Lighting Systems; Technical Note 12; International Commission on Illumination: Vienna, Austria, 2021. [CrossRef]

12. Baker, N.; Joint Polar Satellite System (JPSS); Visible Infrared Imaging Radiometer Suite (VIIRS); Sensor Data Records (SDR); Algorithm Theoretical Basis Document (ATBD). Goddard Space Flight Centre, Greenbelt Maryland, Ground Project Code, 474,474-00053. 22 April 2011. Available online: https://www.star.nesdis.noaa.gov/jpss/documents/ATBD/D0001-M01-S01-00 4_JPSS_ATBD_VIIRS-Geolocation_A.pdf (accessed on 7 March 2022).

13. Elvidge, C.D.; Keith, D.M.; Tuttle, B.T.; Baugh, K.E. Spectral identification of lighting type and character. Sensors 2010, 10, 3961–3988. [CrossRef]

14. Box, G.E.P.; Jenkins, G.M.; Reinsel, G.C. Autocorrelation Function and Spectrum of Stationary Processes. In Time Series Analysis; Wiley Online Library: Hoboken, NJ, USA, 2008; ISBN 9781118619193.

15. Elvidge, C.D.; Hsu, F.-C.; Zhizhin, M.; Ghosh, T.; Taneja, J.; Bazilian, M. Indicators of electric power instability from satellite observed nighttime lights. Remote Sens. 2020, 12, 3194. [CrossRef]

16. Hsu, F.; Zhizhin, M.; Ghosh, T.; Elvidge, C.; Taneja, J. The Annual Cycling of Nighttime Lights in India. Remote Sens. 2021, 13, 1199. [CrossRef]

17. Ord, J.K.; Getis, A. Testing for local spatial autocorrelation in the presence of global autocorrelation. J. Reg. Sci. 2001, 41, 411–432. [CrossRef]

18. The Illuminating Engineering Society of North America. The Lighting Handbook: Reference and Application, 10th ed.; IESNA: New York, NY, USA, 2011; 1328p.

19. Selby, B. The index of dispersion as a test statistic. Biometrika 1965, 52, 627–629. [CrossRef]

20. Kopp, T.J.; Thomas, W.; Heidinger, A.K.; Botambekov, D.; Frey, R.A.; Hutchison, K.D.; Iisager, B.D.; Brueske, K.; Reed, B. The VIIRS Cloud Mask: Progress in the First Year of S-NPP toward a Common Cloud Detection Scheme. J. Geophys. Res. Atmos. 2014, 119, 2441–2456. [CrossRef]

21. Barentine, J.C.; Walker, C.E.; Kocifaj, M.; Kundracik, F.; Juan, A.; Kanemoto, J.; Monrad, C.K. Skyglow changes over Tucson, Arizona, resulting from a municipal LED street lighting conversion. J. Quant. Spectrosc. Radiat. Transf. 2018, 212, 10–23. [CrossRef] 22. Barentine, J.C.; Kundracik, F.; Kocifaj, M.; Sanders, J.C.; Esquerdo, G.A.; Dalton, A.M.; Foott, B.; Grauer, A.; Tucker, S.; Kyba, C.C.M. Recovering the city street lighting fraction from skyglow measurements in a large-scale municipal dimming experiment. J. Quant. Spectrosc. Radiat. Transf. 2020, 253, 107120. [CrossRef]

23. Kyba, C.C.M.; Ruby, A.; Kuechly, H.U.; Kinzey, B.; Miller, N.; Sanders, J.; Barentine, J.; Kleinodt, R.; Espey, B. Direct measurement of the contribution of street lighting to satellite observations of nighttime light emissions from urban areas. Light. Res. Technol. 2021, 53, 189–211. [CrossRef]

24. Dick, R. Chapter 6—Harmonic Oscillators and Coherent States. In Advanced Quantum Mechanics; Springer: Berlin/Heidelberg, Germany, 2012.

25. City of Tucson. HomeGoods Distribution Center Holds Grand Opening. 16 October 2016. Available online: https://www. tucsonaz.gov/ward-5/news/homegoods-distribution-center-holds-grand-opening (accessed on 28 November 2021). 26. Dazhong, S.H.Z. Constant Current Driver for Semiconductor Lighting. Res. Prog. SSE Solid State Electron. 2006, 2. 27. Wang, Z.; Román, M.O.; Kalb, V.L.; Miller, S.D.; Zhang, J.; Shrestha, R.M. Quantifying uncertainties in nighttime light retrievals from Suomi-NPP and NOAA-20 VIIRS Day/Night Band data. Remote Sens. Environ. 2021, 263, 112557. [CrossRef]

Remote Sens. 2022, 14, 1316 33 of 33

28. Ryan, R.E.; Pagnutti, M.; Burch, K.; Leigh, L.; Ruggles, T.; Cao, C.; Aaron, D.; Blonski, S.; Helder, D. The Terra Vega active light source: A first step in a new approach to perform nighttime absolute radiometric calibrations and early results calibrating the VIIRS DNB. Remote Sens. 2019, 11, 710. [CrossRef]

29. Cao, C.; Bai, Y. Quantitative analysis of VIIRS DNB nightlight point source for light power estimation and stability monitoring. Remote Sens. 2014, 6, 11915–11935. [CrossRef]

30. Elvidge, C.D.; Baugh, K.; Zhizhin, M.; Hsu, F.C.; Ghosh, T. VIIRS night-time lights. Int. J. Remote Sens. 2017, 38, 5860–5879. [CrossRef]

31. Elvidge, C.D.; Zhizhin, M.; Ghosh, T.; Hsu, F.-C.; Taneja, J. Annual time series of global VIIRS nighttime lights derived from monthly averages: 2012 to 2019. Remote Sens. 2021, 13, 922. [CrossRef]

32. Weinold, M. A long overdue end to flicker: The 2020 EU lighting efficiency regulations. Camb. J. Sci. Policy 2020, 1, e0628620899. 33. Bullough, J.D. The blue-light hazard: A review. J. Illum. Eng. Soc. 2000, 29, 6–14. [CrossRef]

34. Bullough, J.D.; Bierman, A.; Rea, M.S. Evaluating the blue-light hazard from solid state lighting. Int. J. Occup. Saf. Ergon. 2019, 25, 311–320. [CrossRef]